톤 할당

(54)

1) 기능

시스템에서 제공하는 톤(Tone) 소스를 확인하고 변경한다.

2) 화면

[톤 항목]

Tone Port:##/## ID:##

3) 데이터 설명

- ##/##  : 톤 항목에 할당된 톤 소스가 표시되며 원하는 소스로 변경한다.

- ID:##  : 톤 항목의 순서를 나타내면 변경할 수 없다.

4) 프로그램 방법

54 +  VOL  (원하는 항목 선택) + 톤 소스(포트) 지정 +  보류/입력

5) 관련 DB

6) 보충 설명

- 시스템에서 제공하는 톤 소스는 다음과 같다.

| 톤 소스 | 톤 내용 | 톤 소스 | 톤 내용 |
| --- | --- | --- | --- |
| 00/01 | Melody | 00/02 | Dial |
| 00/03 | Congestion | 00/04 | Busy |
| 00/05 | R.B.T | 00/06 | I.R.B.T |
| 00/07 | Call Wait | 00/08 | Special Dial (Second) |
| 00/09 | Break-In(Intrusion) | 00/10 | Conference |
| 00/11 | Message Wait | 00/12 | Howling |
| 00/13 | Reserved | 00/14 | Reserved |
| 00/15 | Reserved | 00/16 | 54 Tone |

- 기본값으로 기능 수행 중 송출되는 톤은 다음과 같다.

| Tone Item | ID | Tone Source |
| --- | --- | --- |
| EXT First Dial Tone | 01 | 00/02 |
| TRK First Dial Tone | 02 | 00/02 |
| MSG WAT REG Tone (E) | 03 | 00/07 |
| DND REG Tone (EXT) | 04 | 00/07 |
| Call Forward REG (E) | 05 | 00/07 |
| Reorder Dial Tone | 06 | 00/08 |
| EXT Busy Tone | 07 | 00/04 |
| O/G TRK All BSY Tone | 08 | 00/04 |
| IRBT TRK to EXT Call | 09 | 00/06 |
| IRBT EXT to EXT Call | 10 | 00/06 |
| RBT TRK to EXT Call | 11 | 00/05 |
| RBT EXT to EXT Call | 12 | 00/05 |
| RBT ACR EXT Call | 13 | 00/05 |
| RBT Recall Service | 14 | 00/05 |
| Blocking SRVC Tone | 15 | 00/03 |
| DND Service Tone | 16 | 00/11 |
| Uncomplete Dial Tone | 17 | 00/03 |
| Invalid Dialing Tone | 18 | 00/03 |
| Out of Order Tone | 19 | 00/03 |
| Absence Service Tone | 20 | 00/11 |
| Line Lock-out Tone | 21 | 00/12 |
| Vacant Service Tone | 22 | 00/03 |
| Break-in SRVC Tone | 23 | 00/09 |
| Warning Service Tone | 24 | 00/11 |
| Wake-up Service Tone | 25 | 00/01 |
| Camp-on Service Tone | 26 | 00/07 |
| Conference SRVC Tone | 27 | 00/10 |
| EXT Split/Hold (ATD) | 28 | 00/01 |
| TRK Split/Hold (ATD) | 29 | 00/01 |
| Camp-on HLD Tone (T) | 30 | 00/01 |
| Camp-on HLD Tone (E) | 31 | 00/01 |
| Call-wait Hold (T) | 32 | 00/01 |
| Call-wait Hold (E) | 33 | 00/01 |
| STA Holding Tone (T) | 34 | 00/01 |
| STA Holding Tone (E) | 35 | 00/01 |
| Call-park Hold (T) | 36 | 00/01 |
| Call-park Hold (E) | 37 | 00/01 |
| Paging Holding (TRK) | 38 | 00/01 |
| Paging Holding (EXT) | 39 | 00/01 |
| DOD CONV Restriction | 40 | 00/07 |
| DISA Dial Tone | 41 | 00/02 |
| TRK Waiting Tone (A) | 42 | 00/03 |
| DISA Congestion Tone | 43 | 00/03 |
| CONFER Break-in High | 44 | 00/10 |
| CONFER Break-in Low | 45 | 00/10 |
| Warm Line Dial Tone | 46 | 00/08 |
| Admin ERR SRVC Tone | 47 | 00/07 |
| Reserved | 48 | 00/01 |
| E1/T1 AIS Test Tone | 49 | 00/16 |
| Trk Long CNV Warning! | 50 | 00/07 |
| KD/E, LKD series ring | 51 | 00/06 |
| LCR Virtual DIALTone | 52 | 00/08 |
| ACD First Announce | 53 | 00/01 |
| ACD Second Announce | 54 | 00/01 |

기능 코드

(55)

1) 기능

시스템에서 사용중인 기능코드를 확인, 변경한다.

2) 화면

[기능 코드 항목]

Feature Code:???? ID:###

3) 데이터 설명

- ???? : 해당 기능 코드가 표시되면 변경할 수 있다.

- ID:#### : 기능 코드 항목 순서로 바꿀 수 없다

4) 프로그램 방법

55 +  VOL  (원하는 항목 선택)+ 기능 코드 지정 + 보류/입력

5) 관련 DB

6) 보충 설명

- 기능코드 내용을 삭제할 경우에는 거부/착신전환 버튼을 누른다.

다른 Admin.항목과 같이 #을 누르면 입력된 #이 기능코드가 되므로 주의한다.

- 여러 개의 공동단출 다이알 기능코드를 지정할 경우에는 데이터 입력 후 프로그램 버튼을 누르면 다시 공동단출 다이알 기능코드를 지정할 수 있다.

- 다음의 표는 시스템에서 제공하는 기능 코드들이다.

| Function | ID | Code |
| --- | --- | --- |
| Attendant Calling Code | 03 | 0 |
| EXTR-Paging Access Code | 06 | 사용자 |
| Common Speed Dial Code | 08 | 6XX |
| MAN/SEC Calling Code | 09 | *0 |
| Emergency Numbering Code | 10 | 사용자 |
| Loud_Bell Pick-Up Code | 11 | #0 |
| Call-Back Cancel Code | 12 | *55 |
| DND Register Code | 13 | *50 |
| DND Cancel Code | 14 | *51 |
| Call FRWD Cancel Code | 15 | *520 |
| Wake-Up Cancel Code | 16 | #3 |
| Call Pick-Up Code | 17 | ** |
| Broker's Calling Code | 18 | *48 |
| Save & Repeat REG/Answer | 19 | #9 |
| Conference Service Code | 21 | #5 |
| IND Call-Park REG/Answer | 24 | #8 |
| SYS Call-Park REG/Answer | 26 | #7 |
| MAN to MAN Calling Code | 28 | *1 |
| Consultation TRANS Code | 29 | *53 |
| Call Pick-Up (Other GRP) | 30 | *22 |
| Intercept Service Code | 31 | *60 |
| EXTR-Paging Pick-Up Code | 32 | 사용자 |
| Remote DOD Service Code | 33 | *78 |
| Password Modify Code | 34 | *43 |
| CHG Class to Night Code | 35 | *41 |
| CHG Class to Day Code | 36 | *42 |
| CHG I/C Mode to NGT Code | 37 | *61 |
| CHG I/C Mode to DAY Code | 38 | *62 |
| O/G Call Allow Code | 39 | *44 |
| O/G Call Deny Code | 40 | *45 |
| Privacy Register Code | 41 | *461 |
| Wake-Up Register Code | 43 | *3 |
| Group Calling Code | 48 | *47 |
| Absence Information REG | 49 | *8 |
| Message Waiting Register | 50 | *71 |
| Message Waiting Answer | 51 | *72 |
| Message Waiting Cancel | 52 | *70 |
| Camp-On Register | 53 | 5(주1) |
| Camp-On TRANS Register | 54 | *76 |
| IND Speed-Dial REG Code | 55 | *# |
| IND Speed-Dial Use Code | 56 | #6 |
| Call_FRWD (All Call) REG | 57 | *521 |
| Call_FRWD (At Busy) REG | 58 | *522 |
| Call_FRWD (No ANSWR) REG | 59 | *523 |
| Call_FRWD (Remote) REG | 60 | *527 |
| Call_FRWD (Out Side) REG | 61 | *524 |
| Last Number Redial Code | 62 | ## |
| Call-Back Service Code | 64 | 4(주1) |
| Group Hunting Code | 65 | 2(주1) |
| Call-Waiting Register | 66 | *(주1) |
| Override Service Code | 67 | 3(주1) |
| Account Service Code | 80 | #4 |
| Background Music Code | 81 | #2 |
| C-FWD (No ANSR/Busy) REG | 82 | *525 |
| Call-FRWD (Absence) REG | 83 | *526 |
| TRK Hook-Flash Send Code | 84 | #* |
| INTR-Paging Request Code | 85 | *90 |
| INTR-Paging Answer Code | 86 | *49 |
| Mini-Bar Service Code | 88 | *54 |
| Room-Status Service Code | 89 | *56 |
| AAM Mode Change Code | 90 | *57 |
| Door Phone Service Code | 91 | *58 |
| Select DND Reg. Code | 92 | *59 |
| Recording Code | 93 | *63 |
| VPC Recording Code | 94 | *73 |
| CLIP Use Register Code | 95 | *95 |
| CNIP Use Register Code | 96 | *96 |
| Function Cancel | 97 | *99 |
| Indexed Call Pickup Code | 98 | #1 |
| Agent ID Reg./Can. Code | 99 | *98 |
| Authorization Service CD | 100 | *92 |
| Author Code Password REG | 101 | *93 |