# ByVac Terminal

# Version 1.0

## Contents

ByVac Terminal	1

Version 1.0	1

Contents	1

Introduction	1

Installation	1

Settings	3

ICP (In-Circuit Programming)	3

Text Transfer Settings	3

Loading a BASIC Program	4

Loading a Hex File	6

In-Circuit Programming (ICP)	6

Entering into ICP mode	6

EEPROM Space	6

Code Space	8

Downloading TCB into Code Space	8

Support	9

## Introduction

Latest version 1.1.8202.

This is a simple terminal emulator with a few added protocols that makes life easier for Tiny Control Basic (TCB) users. It also has the capability of in-circuit programming the SWK (Swiss Army Knife Project).

## Installation

Unzip the ByVac zip file to a temporary folder and double click the setup.exe file. This will take you through a standard Windows install. To uninstall the program use the uninstall utility from the control panel “Add and Remove Programs”.

Re-size the application and click on the properties icon as Figure 1.

Properties Icon

Figure 1 Initial ByVac Terminal

Select the properties icon and choose a suitable settings and a comm port that exists. The red button to the right of the window will open the comm port. If the comm port does not exist you will not be able to progress any further. See Figure 2.

The screen shot shows the typical settings with perhaps the exception of the comm port which happens to be comm. 7 on this system.

## Settings

Figure 2 Settings dialog

The settings are fairly standard for most terminals. The only items different are the text transfer settings and this is to do with the Tiny Control basic (TCB) loading protocol. Use the settings as shown in Figure 2 and the examples given below should work okay.

### ICP (In-Circuit Programming)

This setting must be found by trial and error. If the ICP feature does not work then check this box. In general if you are using USB then this should not be checked, but this is not always the case.

### Text Transfer Settings

Text transfer settings are used exclusively with TCB. TCB can recognise this particular text transfer protocol and act on it.

Text transfers are carried out line by line. When the ‘End of line wait for character’ box is checked the terminal will wait to receive this character before sending the next line. In the above case it will wait for the ASCII ‘ACK’ character (6) to be received which is the default setting for TCB.

The ‘End of line wait delay ms’ is there for devices that don’t have the end of line wait character protocol. This is a simple delay after sending each line. The reason for a delay or waiting for a character is that it takes a certain amount of time to process the line before another one can be accepted.

The settings in the box give you the opportunity to send a command to the device before starting the text transfer. In the above example ‘LOADB’ is sent to the device whenever a file with an extension of ‘tcb’ is sent. Using TCB this will initiate the BASIC file transfer.

You can of course do this manually by typing LOADB before every text transfer, this saves you the trouble.

## Loading a BASIC Program

This is an example of how to load a BASIC program called test.tcb into the EEPROM space.

The program is very simple and consist of:

10 Print "This is Test"

20 a = 1

30 Print "The value of a is ", a

end

Note the ‘end’ without a line number this tells the file transfer and TCB that there are no more lines to send. You are not likely to have this file so cut and pase the above into Notepad and save it with a ‘.tcb’ extension. Be careful, have a look at the file using file explorer and make sure its not called ‘test.tcb.txt’. Notepad has a nasty habit of appending ‘.txt’ to all its files.

- Set up the terminal

Make sure you are using the correct comm port, click on the red connect icon, this will turn green, reset the microprocessor device and press the space bar. This will initialise TCB to the baud rate you have set in the properties dialog.

Figure 3 Starting the device

- Change to EEPROM space by typing in ROM

The ‘e>’ prompt indicates that you are in the correct space, it also reports how much there is of it. 2k bytes in our case.

Figure 4 Changing to EEPROM space

- Load the basic program called “test.tcb”

Use File>Transmit text file or the forth icon along from the left, Figure 5. Provided you have checked “at the start of transfer send LOADB” in the properties dialog as Figure 2 the file should load an you will be able to run it as Figure 6.

Figure 5 Transmit text file

Figure 6 Running test.tcb

Notice now that the fifth icon is now available which means that at any time you can re-transmit the file without having to re-select it again. The caption at the top of the terminal has also changed to the latest file transmitted (loaded). In this case “test.tcb”.

## Loading a Hex File

Hex files are loaded in exactly the same way but your device MUST have external RAM in order to do this. The only difference being is that LOADH is used prior to the text transfer instead of LOADB. This will tell TCB to expect a hex file and correctly load it into the proper memory space.

## In-Circuit Programming (ICP)

ByVac Terminal is capable of in-circuit programming the SWK project. To do this the terminal goes into a console mode. The following example will show you how to us this interface.

### Entering into ICP mode

To do this you must be connected (the right hand side icon must be green). Following on from the last example use the ‘101’ icon and you should see the screen terminal change to black and M,P,D,V,X,H,? displayed. These are the commands that are available in this mode. To see what they mean press ? at the keyboard, see Figure 7.

NOTE the mouse and menu / icon system is not used in this mode, to exit the mode use ‘X’ at the keyboard.

### EEPROM Space

By default the console works within the data memory or EEPROM space so if you now press V followed by 0000 followed by 0010, you should see part of your basic program ‘Print “This is”.

The command will enable you to view the memory space from one address to the other, the address MUST be specified by four digits.

Figure 7 Console mode displaying help

Another option is to use the modify command ‘M’ here we will change the word “This” into “Tiis”. To do this enter at the keyboard the following M 000a 69 . You can see the results by using the V command again. The Modify command has some sub commands which are in fact displayed when you go into the command. These are:

N – will go to the next memory address

P – will go to the previous memory address

. – (dot) will finish the Modify command and return you back to the main menu

nn – any valid hex number will change the byte at that address

CR – will re-fetch the byte at that address

Figure 8 Modifying memory contents

### Code Space

Now enter P to select the flash memory area or the Code or Program space. You will notice that the text becomes yellow indicating that anything we now do will be in this space. To prove this use the view command to view the memory contents between 0080 and 00a0. You should see part of the TCB sign on message (assuming you have programmed the chip with TCB).

### Downloading TCB into Code Space

One of the most interesting areas of using this system is that you can download into code space anything you like. TCB is very convenient because it gives you a ‘platform’ on which to work. To download TCB the file is called TCB-X.hex. Make sure you are in program space (yellow) use the H command and select TCB-X.hex file. This will take a fare amount of time so be patient. You can see the bytes being programmed as it progresses.

Finally press X to exit from the console mode, Figure 9. Try running the basic program again has the ‘h’ from ‘This’ been turned into an ‘i’?

Figure 9 Exit console mode

## Support

This version of ByVac Terminal is not supported, however I can be contacted at jim@byvac.com