CH.2 Answers to Questions

2.1	In a stored program computer, programs are represented in a form suitable for storing in memory alongside the data. The computer gets its instructions by reading them from memory, and a program can be set or altered by setting the values of a portion of memory.

2.2	A main memory, which stores both data and instructions: an arithmetic and logic unit (ALU) capable of operating on binary data; a control unit, which interprets the instructions in memory and causes them to be executed; and input and output (I/O) equipment operated by the control unit.

2.3	Gates, memory cells, and interconnections among gates and memory cells.

2.4	Moore observed that the number of transistors that could be put on a single chip was doubling every year and correctly predicted that this pace would continue into the near future.

2.5	Similar or identical instruction set: In many cases, the same set of machine instructions is supported on all members of the family. Thus, a program that executes on one machine will also execute on any other. Similar or identical operating system: The same basic operating system is available for all family members. Increasing speed: The rate of instruction execution increases in going from lower to higher family members. Increasing Number of I/O ports: In going from lower to higher family members. Increasing memory size: In going from lower to higher family members. Increasing cost: In going from lower to higher family members.

2.6	In a microprocessor, all of the components of the CPU are on a single chip.

Answers to Problems

2.5	All data paths to/from MBR are 40 bits. All data paths to/from MAR are 12 bits. Paths to/from AC are 40 bits. Paths to/from MQ are 40 bits.

2.8	As noted in the answer to Problem 2.7, even though the Intel machine may have a faster clock speed (2.4 GHz vs. 1.2 GHz), that does not necessarily mean the system will perform faster. Different systems are not comparable on clock speed. Other factors such as the system components (memory, buses, architecture) and the instruction sets must also be taken into account. A more accurate measure is to run both systems on a benchmark. Benchmark programs exist for certain tasks, such as running office applications, performing floating point operations, graphics operations, and so on. The systems can be compared to each other on how long they take to complete these tasks. According to Apple Computer, the G4 is comparable or better than a higher-clock speed Pentium on many benchmarks.

2.10

|  | Ic 명령어 수 | P 명령어 해독하고 실행하는데 필요한 사이클 수 | M 필요한 기억장치 참조의 수 | K 기억장치 사이클 시간과 프로세서 사이클 시간의 비율 |  =1/f 고정된 사이클 시간 |
| --- | --- | --- | --- | --- | --- |
| Instruction set architecture | X | X |  |  |  |
| Compiler technology | X | X | X |  |  |
| Processor implementation |  | X |  |  | X |
| Cache and memory hierarchy |  |  |  | X | X |

Source: [HWAN93]

2.11	MIPS rate = f/(CPI  106)

2.13	CPI = 1.55; MIPS rate = 25.8; Execution time = 3.87 ns. Source: [HWAN93]

CH.3 Answers to Questions

3.1	Processor-memory: Data may be transferred from processor to memory or from memory to processor. Processor-I/O: Data may be transferred to or from a peripheral device by transferring between the processor and an I/O module. Data processing: The processor may perform some arithmetic or logic operation on data. Control: An instruction may specify that the sequence of execution be altered.

3.2	Instruction address calculation (iac): Determine the address of the next instruction to be executed. Instruction fetch (if): Read instruction from its memory location into the processor. Instruction operation decoding (iod): Analyze instruction to determine type of operation to be performed and operand(s) to be used. Operand address calculation (oac): If the operation involves reference to an operand in memory or available via I/O, then determine the address of the operand. Operand fetch (of): Fetch the operand from memory or read it in from I/O. Data operation (do): Perform the operation indicated in the instruction. Operand store (os): Write the result into memory or out to I/O.

3.3	(1) Disable all interrupts while an interrupt is being processed. (2) Define priorities for interrupts and to allow an interrupt of higher priority to cause a lower-priority interrupt handler to be interrupted.

3.4	Memory to processor: The processor reads an instruction or a unit of data from memory. Processor to memory: The processor writes a unit of data to memory. I/O to processor: The processor reads data from an I/O device via an I/O module. Processor to I/O: The processor sends data to the I/O device. I/O to or from memory: For these two cases, an I/O module is allowed to exchange data directly with memory, without going through the processor, using direct memory access (DMA).

3.5	With multiple buses, there are fewer devices per bus. This (1) reduces propagation delay, because each bus can be shorter, and (2) reduces bottleneck effects.

3.6	System pins: Include the clock and reset pins. Address and data pins: Include 32 lines that are time multiplexed for addresses and data. Interface control pins: Control the timing of transactions and provide coordination among initiators and targets. Arbitration pins: Unlike the other PCI signal lines, these are not shared lines. Rather, each PCI master has its own pair of arbitration lines that connect it directly to the PCI bus arbiter. Error Reporting pins: Used to report parity and other errors. Interrupt Pins: These are provided for PCI devices that must generate requests for service. Cache support pins: These pins are needed to support a memory on PCI that can be cached in the processor or another device.  64-bit Bus extension pins: Include 32 lines that are time multiplexed for addresses and data and that are combined with the mandatory address/data lines to form a 64-bit address/data bus. JTAG/Boundary Scan Pins: These signal lines support testing procedures defined in IEEE Standard 1149.1.

Answers to Problems

3.1	Memory (contents in hex): 300: 3005;   301: 5940;   302: 7006

Step 1:  3005  IR;    Step 2: 3  AC

Step 3:  5940  IR;    Step 4: 3 + 2 = 5  AC

Step 5:  7006  IR;    Step 6: AC  Device 6

3.2	1.	a.	The PC contains 300, the address of the first instruction. This value is loaded in to the MAR.

b.	The value in location 300 (which is the instruction with the value 1940 in hexadecimal)  is loaded into the MBR, and the PC is incremented. These two steps can be done in parallel.

c.	The value in the MBR is loaded into the IR.

2.	a.	The address portion of the IR (940) is loaded into the MAR.

b.	The value in location 940 is loaded into the MBR.

c.	The value in the MBR is loaded into the AC.

3.	a.	The value in the PC (301) is loaded in to the MAR.

b.	The value in location 301 (which is the instruction with the value 5941)  is loaded into the MBR, and the PC is incremented.

c.	The value in the MBR is loaded into the IR.

4.	a.	The address portion of the IR (941) is loaded into the MAR.

b.	The value in location 941 is loaded into the MBR.

c.	The old value of the AC and the value of location MBR are added and the result is stored in the AC.

5.	a.	The value in the PC (302) is loaded in to the MAR.

b.	The value in location 302 (which is the instruction with the value 2941)  is loaded into the MBR, and the PC is incremented.

c.	The value in the MBR is loaded into the IR.

6.	a.	The address portion of the IR (941) is loaded into the MAR.

b.	The value in the AC is loaded into the MBR.

c.	The value in the MBR is stored in location 941.

3.5	Clock cycle =

Bus cycle = 4  125 ns = 500 ns

2 bytes transferred every 500 ns; thus transfer rate = 4 MBytes/sec

Doubling the frequency may mean adopting a new chip manufacturing technology (assuming each instructions will have the same number of clock cycles); doubling the external data bus means wider (maybe newer) on-chip data bus drivers/latches and modifications to the bus control logic. In the first case, the speed of the memory chips will also need to double (roughly) not to slow down the microprocessor; in the second case, the "wordlength" of the memory will have to double to be able to send/receive 32-bit quantities.

3.17	Consider a mix of 100 instructions and operands. On average, they consist of 20 32-bit items, 40 16-bit items, and 40 bytes. The number of bus cycles required for the 16-bit microprocessor is (2  20) + 40 + 40 = 120. For the 32-bit microprocessor, the number required is 100. This amounts to an improvement of 20/120 or about 17%. Source: [PROT88].

3.19

CH.4 Answers to Questions

4.1	Sequential access: Memory is organized into units of data, called records. Access must be made in a specific linear sequence. Direct access: Individual blocks or records have a unique address based on physical location. Access is accomplished by direct access to reach a general vicinity plus sequential searching, counting, or waiting to reach the final location. Random access: Each addressable location in memory has a unique, physically wired-in addressing mechanism. The time to access a given location is independent of the sequence of prior accesses and is constant.

4.2	Faster access time, greater cost per bit; greater capacity, smaller cost per bit; greater capacity, slower access time.

4.3	It is possible to organize data across a memory hierarchy such that the percentage of accesses to each successively lower level is substantially less than that of the level above. Because memory references tend to cluster, the data in the higher-level memory need not change very often to satisfy memory access requests.

4.4	In a cache system, direct mapping maps each block of main memory into only one possible cache line. Associative mapping permits each main memory block to be loaded into any line of the cache. In set-associative mapping, the cache is divided into a number of sets of cache lines; each main memory block can be mapped into any line in a particular set.

4.5	One field identifies a unique word or byte within a block of main memory. The remaining two fields specify one of the blocks of main memory. These two fields are a line field, which identifies one of the lines of the cache, and a tag field, which identifies one of the blocks that can fit into that line.

4.6	A tag field uniquely identifies a block of main memory. A word field identifies a unique word or byte within a block of main memory.

4.7	One field identifies a unique word or byte within a block of main memory. The remaining two fields specify one of the blocks of main memory. These two fields are a set field, which identifies one of the sets of the cache, and a tag field, which identifies one of the blocks that can fit into that set.

4.8	Spatial locality refers to the tendency of execution to involve a number of memory locations that are clustered. Temporal locality refers to the tendency for a processor to access memory locations that have been used recently.

4.9	Spatial locality is generally exploited by using larger cache blocks and by incorporating prefetching mechanisms (fetching items of anticipated use) into the cache control logic. Temporal locality is exploited by keeping recently used instruction and data values in cache memory and by exploiting a cache hierarchy.

Answers to Problems

4.1	The cache is divided into 16 sets of 4 lines each. Therefore, 4 bits are needed to identify the set number. Main memory consists of 4K = 212 blocks. Therefore, the set plus tag lengths must be 12 bits and therefore the tag length is 8 bits. Each block contains 128 words. Therefore, 7 bits are needed to specify the word.

|  | TAG | SET | WORD |
| --- | --- | --- | --- |
| Main memory address = | 8 | 4 | 7 |

4.2	There are a total of 8 kbytes/16 bytes = 512 lines in the cache. Thus the cache consists of 256 sets of 2 lines each. Therefore 8 bits are needed to identify the set number. For the 64-Mbyte main memory, a 26-bit address is needed. Main memory consists of 64-Mbyte/16 bytes = 222 blocks. Therefore, the set plus tag lengths must be 22 bits, so the tag length is 14 bits and the word field length is 4 bits.

|  | TAG | SET | WORD |
| --- | --- | --- | --- |
| Main memory address = | 14 | 8 | 4 |

4.3

| Address | 111111 | 666666 | BBBBBB |
| --- | --- | --- | --- |
| a. Tag/Line/Word | 11/444/1 | 66/1999/2 | BB/2EEE/3 |
| b. Tag /Word | 44444/1 | 199999/2 | 2EEEEE/3 |
| c. Tag/Set/Word | 22/444/1 | CC/1999/2 | 177/EEE/3 |

4.4	a.	Address length: 24; number of addressable units: 224; block size: 4; number of blocks in main memory: 222; number of lines in cache: 214; size of tag: 8.

b.	Address length: 24; number of addressable units: 224; block size: 4; number of blocks in main memory: 222; number of lines in cache: 4000 hex; size of tag: 22.

c.	Address length: 24; number of addressable units: 224; block size: 4; number of blocks in main memory: 222; number of lines in set: 2; number of sets: 213; number of lines in cache: 214; size of tag: 9.

4.8	a.	8 leftmost bits = tag; 5 middle bits = line number; 3 rightmost bits = byte number

b.	slot 3; slot 6; slot 3; slot 21

c.	Bytes with addresses 0001 1010 0001 1000 through 0001 1010 0001 1111 are stored in the cache

d.	256 bytes

e.	Because two items with two different memory addresses can be stored in the same place in the cache.  The tag is used to distinguish between them.

4.11	a.	Address format: Tag = 20 bits; Line = 6 bits; Word = 6 bits

Number of addressable units = 2s+w = 232 bytes; number of blocks in main memory = 2s = 226; number of lines in cache 2r = 26 = 64; size of tag = 20 bits.

b.	Address format: Tag = 26 bits; Word = 6 bits

Number of addressable units = 2s+w = 232 bytes; number of blocks in main memory = 2s = 226; number of lines in cache = undetermined; size of tag = 26 bits.

c.	Address format: Tag = 9 bits; Set = 17 bits; Word = 6 bits

Number of addressable units = 2s+w = 232 bytes; Number of blocks in main memory = 2s = 226; Number of lines in set = k = 4; Number of sets in cache = 2d = 217; Number of lines in cache = k  2d =219; Size of tag = 9 bits.

4.19	There are three cases to consider:

| Location of referenced word | Probability | Total time for access in ns |
| --- | --- | --- |
| In cache | 0.9 | 20 |
| Not in cache, but in main memory | (0.1)(0.6) = 0.06 | 60 + 20 = 80 |
| Not in cache or main memory | (0.1)(0.4) = 0.04 | 12ms + 60 + 20 = 12,000,080 |

So the average access time would be:

Avg = (0.9)(20) + (0.06)(80) + (0.04)(12000080) = 480026 ns

CH.5 Answers to Questions

5.1	They exhibit two stable (or semistable) states, which can be used to represent binary 1 and 0; they are capable of being written into (at least once), to set the state; they are capable of being read to sense the state.

5.2	(1) A memory in which individual words of memory are directly accessed through wired-in addressing logic. (2) Semiconductor main memory in which it is possible both to read data from the memory and to write new data into the memory easily and rapidly.

5.3	SRAM is used for cache memory (both on and off chip), and DRAM is used for main memory.

5.4	SRAMs generally have faster access times than DRAMs. DRAMS are less expensive and smaller than SRAMs.

5.5	A DRAM cell is essentially an analog device using a capacitor; the capacitor can store any charge value within a range; a threshold value determines whether the charge is interpreted as 1 or 0. A SRAM cell is a digital device, in which binary values are stored using traditional flip-flop logic-gate configurations.

5.6	Microprogrammed control unit memory; library subroutines for frequently wanted functions; system programs; function tables.

5.7	EPROM is read and written electrically; before a write operation, all the storage cells must be erased to the same initial state by exposure of the packaged chip to ultraviolet radiation. Erasure is performed by shining an intense ultraviolet light through a window that is designed into the memory chip. EEPROM  is a read-mostly memory that can be written into at any time without erasing prior contents; only the byte or bytes addressed are updated. Flash memory is intermediate between EPROM and EEPROM in both cost and functionality. Like EEPROM, flash memory uses an electrical erasing technology. An entire flash memory can be erased in one or a few seconds, which is much faster than EPROM. In addition, it is possible to erase just blocks of memory rather than an entire chip. However, flash memory does not provide byte-level erasure. Like EPROM, flash memory uses only one transistor per bit, and so achieves the high density (compared with EEPROM) of EPROM.

5.8	A0 - A1 = address lines:. CAS = column address select:. D1 - D4 = data lines. NC: = no connect. OE: output enable. RAS = row address select:. Vcc: = voltage source. Vss: = ground. WE: write enable.

5.9	A bit appended to an array of binary digits to make the sum of all the binary digits, including the parity bit, always odd (odd parity) or always even (even parity).

5.10	A syndrome is created by the XOR of the code in a word with a calculated version of that code. Each bit of the syndrome is 0 or 1 according to if there is or is not a match in that bit position for the two inputs. If the syndrome contains all 0s, no error has been detected. If the syndrome contains one and only one bit set to 1, then an error has occurred in one of the 4 check bits. No correction is needed. If the syndrome contains more than one bit set to 1, then the numerical value of the syndrome indicates the position of the data bit in error. This data bit is inverted for correction.

5.11	Unlike the traditional DRAM, which is asynchronous, the SDRAM exchanges data with the processor synchronized to an external clock signal and running at the full speed of the processor/memory bus without imposing wait states.

Answers to Problems

5.2	In 1 ms, the time devoted to refresh is 64  150 ns = 9600 ns. The fraction of time devoted to memory refresh is (9.6 10–6 s)/10–3 s = 0.0096, which is approximately 1%.

5.3	a.	Memory cycle time = 60 + 40 = 100 ns. The maximum data rate is 1 bit every 100 ns, which is 10 Mbps.

b.	320 Mbps = 40 MB/s.

5.9	Total memory is 1 megabyte = 8 megabits. It will take 32 DRAMs to construct the memory (32  256 Kb = 8 Mb). The composite failure rate is 2000  32 = 64,000 FITS. From this, we get a MTBF = 109/64,000 = 15625 hours = 22 months. Source: [PROT88].

5.10	The stored word is 001101001111, as shown in Figure 5.10. Now suppose that the only error is in C8, so that the fetched word is 001111001111. Then the received block results in the following table:

| Position | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits | D8 | D7 | D6 | D5 | C8 | D4 | D3 | D2 | C4 | D1 | C2 | C1 |
| Block | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 |
| Codes |  |  | 1010 | 1001 |  | 0111 |  |  |  | 0011 |  |  |

The check bit calculation after reception:

| Position | Code |
| --- | --- |
| Hamming | 1111 |
| 10 | 1010 |
| 9 | 1001 |
| 7 | 0111 |
| 3 | 0011 |
| XOR = syndrome | 1000 |

The nonzero result detects and error and indicates that the error is in bit position 8, which is check bit C8.

5.11	Data bits with value 1 are in bit positions 12, 11, 5, 4, 2, and 1:

| Position | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits | D8 | D7 | D6 | D5 | C8 | D4 | D3 | D2 | C4 | D1 | C2 | C1 |
| Block | 1 | 1 | 0 | 0 |  | 0 | 0 | 1 |  | 0 |  |  |
| Codes | 1100 | 1011 |  |  |  |  |  | 0101 |  |  |  |  |

The check bits are in bit numbers 8, 4, 2, and 1.

Check bit 8 calculated by values in bit numbers: 12, 11, 10 and 9

Check bit 4 calculated by values in bit numbers: 12, 7, 6, and 5

Check bit 2 calculated by values in bit numbers: 11, 10, 7, 6 and 3

Check bit 1 calculated by values in bit numbers: 11, 9, 7, 5 and 3

Thus, the check bits are:  0 0 1 0

5.12	The Hamming Word initially calculated was:

| bit number: | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 |

Doing an exclusive-OR of 0111 and 1101 yields 1010 indicating an error in bit 10 of the Hamming Word.  Thus, the data word read from memory was 00011001.