QMOUSE.DOC

******************************

Programmer's Reference Manual

of  Quick Mouse Driver

Serial Mouse Version

Release 7.0

******************************

Introduction

This  document contains the following information of QUICK  MOUSE

Driver:

mouse functions,

text cursors,

and graphics curcor,

for  programmers to support their application programs with QUICK

MOUSE Driver.

This  protocol is compatible with the protocol in "Microsoft  (R)

Mouse	User's   Guide  for  IBM  (R)  Personal  Computers"   for

installation and operation.

Contents

***********************************************************

Calling Mouse Functions in Assembly Language

Calling Mouse Functions in Basic Language

Calling Mouse Functions in Pascal Language

Notice to System Programmers

Functions of QUICK MOUSE

APPENDIX

The Cursors QUICK MOUSE provides

(1) Text cursors

a. Hardware text cursor

b. Software text cursor

(2) Graphics cursors

***********************************************************

The  programming interface uses the 8086,8088 software	interrupt

51.  The  registers  pass  and return the  parameter values. This

interface can be used in from assembly language programs to  high

level language programs such as BASIC,	COBOL, FORTRAN, PASCAL or

C.

Calling Mouse Functions in Assembly Language :

Do as the following  steps:

1. Load  the AX,  BX,	CX,and Dx registers with  the  parameter

values.

2. Execute software interrupt 51(33H).

Example

To set the minimum & maxmum X position to 100 (minimum) and  300

(maximum):

;	set the minimum & maxmum X position to (100,300)

Mov  AX,7	   ;function 7

Mov  CX,100   ;set Minimum X to 100

Mov  DX,300   ;set Maximum X to 300

Int  51	   ;interrupt to mouse

Note:  In  Assembly Language,  calling function 9 and 12 of mouse

system  calls  expect  somewhat different values  for  the

forth parameter than calling in a Basic program.

Calling Mouse Functions in BASIC (BASICA) Language :

First, you must place the following statements:

010	DEF SEG=0

020	MSEG=256*PEEK(51*4+3)+PEEK(51*4+2)

030	MOUSE=256*PEEK(51*4+1)+PEEK(51*4)+2

040	DEF SEG=MSEG

Then

CALL MOUSE(M1%,M2%,M3%,M4%)

M1%, M2%, M3%, and M4% are integer variables you have chosen for

parameters  in this call,  which correspond to the values for AX,

BX, CX, and DX registers seperatively.

For example:

100  ;Set minimum and maximum X position to (100,300)

200  M1%=7	 'function 7

300  M3%=100 'minimum X

400  M4%=300 'maximum X

500  CALL MOUSE(M1%,M2%,M3%,M4%)

Calling Mouse Functions in Pascal Language:

First, you must place the following subroutine before any call:

PROCEDURE MOUSE(VAR A,B,C,D,E,F:INTEGER);

TYPE MS = RECORD

{ Parameters for Registers }

AX,BX,CX,DX,BP,SI,DI,DS,ES,FLAGS:INTEGER;

END;

VAR VMS:MS;

BEGIN

WITH VMS DO

BEGIN

AX:=A;

BX:=B;

CX:=C;

DX:=D;

SI:=E;

DI:=F;

INTR($33,VMS);  { interrupt 33h }

A:=AX;

B:=BX;

C:=CX;

D:=DX;

END;

END;

Then you can call the mouse functions as:

{ Set minimum and maximum X position to (100,300) }

A:=7;  {function 7}

C:=100;

D:=300;

MOUSE(A,B,C,D,E,F);

{ Define software text cursor that has the shape  correspond

to ASCII Code (2) }

A:=10;	 {function 10}

B:=0;	 {software text cursor}

C:=$7700; {screen mask}

D:=2;	 {cursor mask}

MOUSE(A,B,C,D,E,F);

Notice to System Programmers

When intercepting the video i/o software interrupt (int 10h), you

should be very careful	and note the following:

The  mouse driver guarantees consistency of the  screen  contents

with  programs	that  write  on the screen via int  10h  (or  via

DOS),  and  the program should guarantee that video mode  changes

(function 0) be intercepted by the mouse driver.

You  can  also detect the presence of the mouse by  checking  the

interrupt vector entry for interrupt 51 in low memory for value 0

or  for  pointing  to an IRET  instruction.  As  this  method  is

dependent on the release of DOS, it should be used carefully.

In  some  programs such as GWBASIC write directly  on  the  video

memory, the application program should hide cursor (function 2 or

16)  before writing,  then show cursor(function 1) after writing.

Functions of QUICK MOUSE

QUICK  MOUSE provides users with the following functions,   which

are  described in greater detail in later sections.

FUNCTION NUMBER    FUNCTIONS

***********************************************************

0	  Mouse Initialization

1	  Show Cursor

2	  Hide Cursor

3	  Get Button Status & Cursor Position

4	  Set Cursor Position

5	  Get Button Press Information

6	  Get Button Release Information

7	  Set Horizontal Cursor Range

8	  Set Vertical Cursor Range

10	  Define Text Cursor

11	  Get Mouse Movement Counters

12	  Event Handler

13	  Light Pen Emulation On

14	  Light Pen Emulation Off

15	  Set Mouse Movement/Pixel Ratio

16	  Conditional Off

19	  Set Double-Speed Threshold

***********************************************************

Description of the Mouse Functions

The  programming interface uses the 8086,8088 software	interrupt

51.  The following examples in Pascal for each function are based

on the procedure pre-defined in the above section " Calling Mouse

Functions in Pascal Language."

(0) Mouse Initialization

========================

This  function	shows whether the hardware and software of  mouse

are installed. Th� mouse status is

0 : if the hardware and software of mouse are not installed,

-1 : if the hardware and software of mouse are installed.

Example

input

A:= 0;

MOUSE(A,B,C,D,E,F);

output

A:= mouse status

B:= number of button in use

In Microsoft Mouse Mode, B:=2.

In Mouse System Mouse Mode, B:=3.

(1) Show Cursor

===============

This function increments the internal cursor flag.  The cursor is

displayed  on  the screen if the flag is 0,  and is tracking  the

movement of the mouse.

Example

input

A:= 1;

MOUSE(A,B,C,D,E,F);

output

none (cursor is showed)

(2) Hide Cursor

===============

This  function decrements the internal cursor flag,  and  removes

the  cursor from the screen.  Although hidden,	the cursor  still

tracks	the movement of the mouse.

Example

Input

A:= 2;

MOUSE(A,B,C,D,E,F);

Output

none (cursor is hidden)

(3) Get Button Status & Cursor Position

=======================================

This function reports:

the status of the buttons,

and the position of the cursor.

The button status is a single integer value with:

bit 0 - the left button (L)

bit 1 - the right button (R)

bit 2 - the middle button (M) (only in Mouse System Mode)

And the bits represent in output:  0 - the button is up

1 - the button is down.

So you will get:    BX	 Button down

0	     0

1	  L

2		R

3	  L	R

4	     M

5	  L  M

6	     M	R

7	  L  M	R

Example

input

A:= 3;

MOUSE(A,B,C,D,E,F);

Output

B:= button status

C:= horizontal (X) cursor position

D:= vertical (Y) cursor position

(4) Set Mouse Position

======================

This  function	sets the cursor to the	assigned  horizontal  and

vertical  positions on the screen.  The new values must be within

the permitted ranges of the virtual screen,  so  the values  will

be  rounded  to  the nearest values permitted by the  screen  for

horizontal  and  vertical positions If you are not using  a  high

resolution screen.

Example

Input

A:= 4;

C:= new horizontal (X) cursor position

D:= new vertical (Y) cursor position

MOUSE(A,B,C,D,E,F);

Output

none

(5) Get Button Press Information

================================

This function reports the current button status  and the count of

button	presses  since	the last call  to the same function,  and

position of the cursor last time the button was pressed.

The BX parameter indicates which button was pressed.

BX: 0 - the left button is checked.

1 - the right button is checked.

The button status is a single integer value with:

bit 0 - the left button

bit 1 - the right button,

And the bits represent in output:  0 - the button is up

1 - the button is down.

Example

input

A:= 5;

B:= button to be checked

MOUSE(A,B,C,D,E,F);

Output

A:= button status

B:= number of button presses since the last same call

C:= horizontal cursor position at last press

D:= vertical cursor position at last press

(6) Get Button Release Information

==================================

This function reports the current button status, and the count of

the button releases since the last call to the same function, and

the  position  of the cursor  since the last time  the button was

released.

The BX parameter specifies which button is checked.

BX: 0 - the left button is checked

1 - the right button is checked.

The button status is a single integer value with:

bit 0 - the left button

bit 1 - the right button,

And the bits represent in output:  0 - the button is up

1 - the button is down.

Example

Input

A:= 6;

B:= button to be checked

MOUSE(A,B,C,D,E,F);

Output

A:= button status

B:= number of button releases since the last same call

C:= horizontal cursor position at last release

D:= vertical cursor position at last release

(7) Set Horizontal Cursor Range

===============================

This  function sets the horizontal cursor range of the screen  to

restrict  all  cursor  movement to this area.  If the  cursor  is

outside the defined area when the call is made,  it moves to just

inside the area. If the minimum value is greater than the maximum

the two values will be exchanged.

The virtual screen determines the minimum and the maximum values.

Example

Input

A:= 7;

C:= new minimum horizontal cursor position

D:= new maximum horizontal cursor position

MOUSE(A,B,C,D,E,F);

Output

none

(8) Set Vertical Cursor Range

=============================

This  function	sets the vertical cursor range on the  screen  to

restrict  all  cursor movement to this area.  If  the  cursor  is

outside the defined area when the call is made,  it moves to just

inside the area. If the minimum value is greater than the maximum

the two values will be exchanged.

Th� virtual screen determines the maximum and minimum values.

Example

Input

A:= 8;

C:= new minimum vertical cursor position

D:= new maximum vertical cursor position

MOUSE(A,B,C,D,E,F);

Output

none

(9) Define Graphics Cursor Block

================================

In graphics mode, this function determines the shape and color of

the cursor,  and identifies the center of the cursor.  The cursor

block  is a 16 X 16 bit pattern,  and the cursor hot spot  values

must  be bwithin the range of -16 to 16.  They define  one  pixel

within (or outside) the cursor block.

Example

Input

A:= 9;

B:= horizontal cursor hot spot

C:= vertical cursor hot spot

D:= pointer to screen and cursor mask

MOUSE(A,B,C,D,E,F);

Output

none

(10) Define Text Cursor

=======================

In  text  mode,  this function choose  which of the  software  or

hardware  cursor is present.  The value of the parameter BX  sets

the cursor type:

BX: 0 - the software text cursor

1 - the hardware text cursor

The  values of CX and DX  specify the screen and cursor mask with

the software  cursor, or set  the first and last scan lines to be

shown on the screen with the hardware cursor.

Example

Input

A:= 10;

B:= choose cursor (software or hardware)

C:= screen mask value/ scan line start

D:= cursor mask value/ scan line stop

(software cursor)  (hardware cursor)

MOUSE(A,B,C,D,E,F);

Output

none

(11) Get Mouse Motvement Counters

=================================

This  function gets the horizontal and vertical movement  counts,

which  is the distance the mouse has actually moved in 1/200 inch

(mickey) increments, since the last same call.

The  movement counts are always within the range from  -32768  to

32767.	The positive horizontal count indicates a movement to the

right,	and  the positive vertical count indicates a movement  to

the  bottom  of the screen.  After the call  is  completed,   the

movement count is set to 0.

The  ratio of mouse to cursor movement is variable between  1/200

and 1/400 with the Default Accelerator installed.

�

Example

Input

A:= 11;

MOUSE(A,B,C,D,E,F);

Output

C: horizontal movement count

D: vertical movement count

(12) Define Event Handler

=========================

This  function gives the mouse Software Interrupts the call  mask

and   the subroutine address,  which  will stop the execution  of

your program and call the speified subroutine when one or more of

the conditions defined by the call mask occur.

The  call  mask  is  a	single integer	value  that  defines  the

conditions  which will cause an interrupt.  Each bit in the  call

mask corresponds to a specific condition:

Mask Bit   Condition

0       change cursor position

1       press left button

2       release left button

3       press right button

4       release right button

5-15     not used

Example

Input

A:= 12;

C:= call mask

{ ES:DX = address of subroutine }

D:= address of subroutine

MOUSE(A,B,C,D,E,F);

Output

none

(13) Light Pen Emulation On

===========================

This function sets on light pen emulation by the mouse.  Calls to

the pen function will return  the cursor position at the last pen

down which is controlled by the mouse buttons.	When both buttons

are down,  the pen is down the screen.	When both buttons are up,

the pen is off the screen.

Example

Input

A: =13;

MOUSE(A,B,C,D,E,F);

Output

none

(14) Light Pen Emulation Off

============================

This function sets off the light pen emulation. When emulation is

off,  calls to the pen function return information about the true

light pen only.

Example

Input

A:= 14;

MOUSE(A,B,C,D,E,F);

Output

none

(15) Set Mouse Movement/Pixel Ratio

=================================

This  function	sets the speed ratio of mouse movement to  screen

pixel.

The  horizontal  and vertical ratios specify the amount of  mouse

movement  at 8	pixels with  the values falling within	the range

from 1 to 32767. The default values are:

horizontal -	8 steps to 8 pixels

vertical   - 16 steps to 8 pixels.

They are equivalent to:

horizontal - 3.2 inches of mouse movement

vertical   - 2.0 inches of mouse movement.

Example

Input

A:= 15;

C:= horizontal (step to pixel) ratio

D:= vertical (step to pixel) ratio

MOUSE(A,B,C,D,E,F);

Output

none

(16) Conditional Off

====================

This function define an area on the screen within which the mouse

will be turned off.  To reset the region and turn  the mouse  on,

one can call Function 1 - the Show Cursor Function. This function

guards	a portion of the screen for your program that is about to

update.

Example

Input

A:= 16;

C:= left margin

D:= top margin

E:= right margin   { SI }

F:= bottom margin  { DI }

MOUSE(A,B,C,D,E,F);

Output

none

(19) Set Double-Speed Threshold

===============================

This  function sets the threshold speed  of  mouse  movement  for

doubling the cursor's movement on the screen, and makes the mouse

move  easier  when larger and smaller scale cursor  movements are

required in the same time.

Parameter of DX set  the threshold speed  of  mouse.  The  cursor

mvement  will double in speed if the mouse moves faster than  the

value  of DX.	The threshold speed remains until function 19  is

called	again  or  until the mouse is reset by	function  0.  The

default  value	is 64 mickeys (generally 64 X 1/200  inches)  per

second.

The  speed-doubling feature remains once you turn it on.  One way

to  turn off this feature is calling this function  with  setting

DX   to  a  speed  faster than the mouse can  actully  move  (for

example, 10,000).

Example

Input

A:= 19;

D:= threshold speed in mickeys/second

MOUSE(A,B,C,D,E,F);

Output

none

APPENDIX

The Cursors QUICK MOUSE provides

QUICK  MOUSE  provides  the	hardware  text	cursor,  software

text cursor, and graphics cursor. The three cursors are exclusive

- each	at  one  time,	 and  are  optional  for  the  user.  The

characteristics  of the cursor are determined by Functions 9  and

10.

(1) Text Cursor

a. Hardware text cursor

The	hardware  text	cursor	is  a  moving  block,	sometimes

flashing,  which moves from character to character on the screen.

It  is	actually  placed on the screen by  the	video  controller

itself,  and  is defined by the scan lines of the character  cell

numbered from 0 starting from the top scan line. The actual video

controller  and monitor determine the numbers of scan lines on	a

character cell (see the controller documentation for details).

b. Software text cursor

The software text cursor is a character or character attribute

that  also  moves from character to character on the  screen.  It

replaces  and/or modifies the character cell on the screen  where

it is positioned.

The  behavior  of software text cursor is defined by  two  16-bit

values	- the  screen mask and the cursor mask.  The screen  mask

determines  which of the character attributes are preserved.  The

cursor mask determines	the characteristics of the cursor.

The two values are in following format:

bit	  contents

0 - 7	character code

8 -10	foreground color

11	medium intensity(0) or high intensity (1)

12-14	background color

15	non blinking (0) or blinking (1) character

(2) Graphics Cursor

The graphics cursor is a figure such as a sight mark,  an  arrow,

or  others  moving  on	the screen.  It is  an	array  of  pixels

(normally  16  X  16) defined by the screen mask and  the  cursor

mask, each is an array of bits.

The  screen  mask determines which of the cursor  pixels  present

are  part of the shape (1) or of the background (0) (it is  ANDed

with  the screen contents).   The cursor mask determines which of

the  pixels of the cursors contribute to the color/shape  of  the

cursor (it is XORed with the result of the previous operation).

The  operational  behavior  of	these  bit  arrays  are:

screen mask   cursor mask    resulting screen bit

0		   0		  0

0		   1		  1

1		   0		unchanged

1		   1		inverted

The  graphics  cursor  location here means the	point  that  lies

directly  under  the  cursor's target area,  also named  the  hot

spot,  which  is  the  point  that the	mouse  software  uses  to

determine  the cursor coordinates.  The default hot spot  of  the

graphics cursor is the upper left corner of the cursor block.

*******  THE END  *******