AMODE.EXE          version 1.1

General Description

This tech note contains information on ALAMODE.EXE, a Windows NT-only utility to optimize disk drive performance by configuring the drive’s mode pages.    ALAMODE.EXE was created in response to customer demand for a reliable, easy-to-use mode page editing utility for Windows NT platforms.    Other mode page configuration utilities exist, but all that were tested were found to be unacceptable because they    1) require a specific knowledge of what mode pages should be set to;    2) fail to support a wide variety of drive models, reporting errors on many configuration attempts; and/or 3) are generally difficult to use.    ALAMODE allows the user to instantly optimize the mode pages for either Digital Video or Pre-Press applications, or restore the configuration to the factory-supplied default values.

Q.    What are "mode pages" and how do they affect disk drive performance?

A.    A "mode page" or mode parameter page is a group of related settings or parameters that govern certain aspects of how a drive operates.    Depending on the user’s requirements, mode pages can be configured to affect performance, computability, data integrity, and other characteristics.    For example, mode page settings control such things as error recovery and on-disk cache configuration.    Enabling and properly configuring on-disk caching on write operations can significantly increase performance because the drive can immediately store (or cache) the data received in (faster) volatile memory, tell the host it’s done, free the bus, and then take the time to seek to the proper media location and write the cached data to the (slower) drive media.    Meanwhile, the host can prepare data for the next write command, do other processing, or even perform I/O to other drives on the same bus.

Q.    So why aren’t all drives just optimized at the factory?

A. Like everything else in life, mode page configuration generally involves a trade-off.    If you want greater performance, you may have to sacrifice compatibility (some hosts may not expect or respond well to certain high-performance behaviors of the drive) or even a measure of data safety (if the drive loses power at the exact instant the data is in volatile memory, before it is written to the media, the data can be lost).    Most drives come from the factory set for “maximum compatibility” - being as friendly as possible so they can work with the widest range (lowest common denominator) of hosts and systems safely and reliably right out of the box - which may include disabling some high-performance features of which more intelligent hosts could take advantage.

Q.    What kind of performance improvement can I expect?

A.    Anywhere from none to very great (50% or more), depending on many factors, including: how the mode pages are currently configured, what disk drive model it is, what mode pages it supports (many mode page settings are vendor- and/or model- specific), what your main intended use (application) of the drive is, what host adapter you’re using, whether the drive is part of a RAID group, etc., etc., etc.    In general, however, optimizing mode pages will result in a modest performance improvement.    Don’t expect miracles, but don’t be surprised if the improvement is quite noticeable.

Q.    How do I set mode pages using ALAMODE?

A.    Launch the application, select the SCSI drive whose mode pages you want to optimize, and click the button corresponding to the type of application for which you want to optimize performance; Digital Video or Pre-press; or return the drive to its factory Default settings.    It is then a good idea (although not absolutely necessary) to shutdown and restart the system before continuing to use the drive.    This ensures that the drive will actually behave according to the new settings.    If you plan on both formatting the drive and setting its mode pages, it is recommended that you set the mode pages first.

Q.    What if I’m not doing digital video or pre-press work?

A.    Optimizing the mode pages for either digital video or pre-press will very likely improve performance anyway.    When you boil it down all you’re doing is transferring data between the drive and the system.    And although data is transferred somewhat differently (where it goes on the disk, the number and rate of transfer operations, the size of each individual transfer, etc.) in different applications, many of the same rules apply.    Try optimizing for either setting and see if it helps.

Q.    What drives can I optimize?

A.    Generally, any SCSI disk drive attached to any SCSI host adapter - it need not be an ATTO host adapter.

Q.    What drives can’t I optimize?

A. You CANNOT optimize IDE drives, since they don’t have mode pages.

You CANNOT optimize drives that are striped in an ATTO (and maybe other vendor’s) RAID group.

In this case, optimize the drives BEFORE striping them, or break up the group into individual drives, optimize them, and then rebuild the group.    The second method, however, will destroy any data on the existing RAID volume.

Q.    Will configuring a drive’s mode pages harm data on that drive?

A. No.    Unless you have to break up a RAID group into individual drives to set their mode pages, but even then it is the act of breaking up the group that destroys the data, and not the act of setting the mode pages.    Changing mode page settings is quite safe, and, if anything does go wrong, it can be remedied by setting the drive’s mode pages back to Factory Defaults without losing existing data.

Q.    Do I need to format the drive before or after setting mode pages?

A. No.    However, if you plan on both formatting the drive AND setting its mode pages, it is recommended that you set the mode pages first, then proceed with formatting.    It is also recommended (although not absolutely required) that you shutdown the system and restart after changing the mode page settings.

Q.    How does ALAMODE know what to set the mode pages to?

A. The mode page settings that have the greatest effect on performance for specific application types have been found through testing.    Information on these settinges is stored inside ALAMODE.    ALAMODE combines this information with information from the drive on its Default and Changeable settings, and sets the mode parameters accordingly.

Q.    Can I explicitly configure the mode page settings on a page-by-page basis?

A. No, although that capability is intended for a future (as yet unscheduled) release.

Q.    How do I quantify any performance improvement I may see?

A. Good question.    There is much misinformation in the art of measuring performance in general.    Disk benchmark utilities, while generally a good relative measure of performance, are not absolute;    they transfer data a certain way and time how fast it gets there.    If your application transfers data differently than the benchmark (favoring different areas of the disk, transferring smaller or larger data chunks at one time, etc.), then the benchmark may not accurately predict the performance you see.    The only way you’ll really know that    you’ve improved performance is if you notice that the things you actually do on your computer are actually happening faster.

All that having been said, it usually is the case that an increase in drive performance as reflected in a benchmark utility does indicate an overall increase in drive performance for applications in general.

Conclusion...

If you want to optimize a disk drive’s performance for digital video or pre-press applications through mode page configuration in Windows NT, ALAMODE will do it for you, simply and automatically.

If you have any questions or issues not mentioned in this Tech Note, please call ATTO at 716-691-1999 and ask for Technical Support.