SM32X Test Program and ISP Release Note

SM323&321 Test Program and ISP Release Note:

| Release Date | ISP Version | ISP Check Sum | Test AP Version | Description |
| --- | --- | --- | --- | --- |
| 2004/08/15 | AB:2004-08-15 | AB:0005A0DF | 2004/08/15 | TestAP: modify setting password and Win2k,WinXP time out set to 5 minutes ISP: Fixed the MSI mother board boot problem. |
| 2004/08/23 |  |  | 2004/08/23 | TestAP: modify setting password to “320” |
| 2004/09/03 | AC:2004-09-02 | AC:0002AABB | 2004/09/03 | TestAP: support 323AC ISP:323AC ISP release |
| 2004/09/13 | AC:2004-09-10 | AC:0003730E | 2004/09/10 | TestAP: If enlarge bad block , format build partition table will large than real capacity ISP:fix suspend current Fixed interleave mode copy error |
| 2004/09/20 |  |  | 2004/09/20 | TestAP: Win98 Pretest Flash Manager Fail Win98 driver smi320.sys update to 1.2 Enhance Write Performance |
| 2004/09/24 |  |  | 2004/09/24 | TestAP:323AC If already download ISP , Win98 after Pretest no reset |
| 2004/10/01 | AC:2004-10-01 | AC:00051532 | 2004/10/01 | TestAP: Win98 VIA SIS Pretest Fail ISP: Micro Flash suppor Customer 8 LED option |
| 2004/10/14 | AC:2004-10-14 | AC:0005408E | 2004/10/14 | TestAP:add customer = 1 Led setting ISP: Fix Write Protect Page |
| 2004/10/27 | AC:2004-10-21 AD:2004-10-20 | AC:0006565C AD:0001C09A | 2004/10/27 | TestAP: support ad version ISP: Interleave +2K/P mode Flash , Micron Flash , Win2K performance . |
| 2004/10/28 |  |  | 2004/10/28 | TestAP:open AD autorun support |
| 2004/11/12 |  |  | 2004/11/12 | TestAP: 2004/10/28 version format will hang , fix the problem |
| 2004/11/23 |  |  | 2004/11/23 | TestAP:fix AD after test OK LED no flash |
| 2004/11/26 |  |  | 2004/11/26 | TestAP : support multi device test |
| 2004/12/09 | AD:2004-12-10 | AD:00028F46 | 2004/12/09 | TestAP,ISP : fix 323AD customer=8 LED bug |
| 2004/12/13 |  |  | 2004/12/13 | Add Pretest time “no erase” function , for 323AD |
| 2004/12/17 |  |  | 2004/12/17 | Fix 12/13 bad block number is default value , can’t adjust , |
| 2004/12/24 | 321AB: 2004-12-02 | 321AB:00007BCD | 2004/12/24 | Add 321 support Fix 323AD,321AB Pretest “no erase” bad block increase bug . |
| 2004/12/27 | 323AD: 2004-12-27 | 323AD:0003786A | 2004/12/27 | TestAP: open auto run make 323ADISP:fix Win98,ME auto run hang |
| 2004/12/30 |  |  | 2004/12/30 | TestAP: If select make auto run can’t format , Fix win98 , WinXP is OS limit |
| 2005/01/27 | 321AB: 2005-01-27 | 321AB:0008612 | E0127 | TestAP: multi-device after show capacity ISP : fix 321AB customer=8 LED bug |
| 2005/01/27 |  |  | E0127 V2 | TestAP: Add 321AC support |
| 2005/02/02 |  |  | E0202 | TestAP:Add 321AC Support 0xD3,0xD5 Device ID |
| 2005/02/03 | 321AC: 2005-02-03 | 321AC:0007D5D | E0202 | ISP: 321AC ISP fix memorybar plug-in login to security |
| 2005/02/16 |  |  | E0216 | TestAP: 1.Pretest reference flash bad block 2.Auto Clock |
| 2005/02/17 | 323AD: 2005-02-17 | 323AD:0003ABCE |  | ISP:323AD ISP fix Win2k autorun |
| 2005/02/18 |  |  | E0218 | TestAP: Fix 321AC test result LED no flash (for win98) 321AC K9G4G format problem |
| 2005/02/24 |  |  | E0224 | TestAP:321AB 0x185 set to 0 |
| 2005/03/03 |  |  | E0304 | TestAP: modify customer=8 led frequency |
| 2005/03/08 |  |  | E0308 | TestAP: disable cache program for HT/Hynix flash Fix 321AC pretest “No erase” but bad block number reduce problem |
| 2005/03/08 V2 |  |  | E0308 V2 | TestAP : enable cache program for HT/Hynix flash |
| 2005/03/15 | 323AD: 2005-03-15 | 323ADISP :0003AD79 |  | 323ADISP: Fix Mac OS 10.3.6 ZIP type show error message |
| 2005/03/23 | 321AC: 2005-03-03 | 321ACISP:0001E576 | E0323 V2 | TestAP: option 0x185 set to 0 , detect AGAND/MLC flash set min bad block to 44 ST/Hynix clock set to 2 321AC ISP : Fix Winxp,Win2k AutoRun |
| 2005/03/29 | 323AD_SEQ: 2005-03-25 | 323ADISP_SEQ: 0001F024 | E0329 | Support ST/Hynix 2K flash Samsung K9F4G,K9K8G,K9L8G |
| 2005/03/30 | 323AD_SEQ: 2005-03-29 | 323ADISP_SEQ: 0001F5D1 | E0330 | Fix ST/Hynix twin mode |
| 2005/04/01 | 323AD_SEQ: 2005-03-31 | 323ADISP_SEQ: 0003B5E9 | E0401 | ST/Hynix set to 20MHz Fix ST/Hynix twin mode |
| 2005/04/04 |  |  | E0404 | ST/Hynix 2K set to Read 30 Write 20 |
| 2005/04/07 |  |  | E0407 | Samaung K9K4G change to R/W 40MHz |
| 2005/04/07 |  |  | E0407 V2 | Add support K9L8G twin mode Add Led Ready Led busy setting |
| 2005/04/09 |  |  | E0409 | Add Flash Database file |
| 2005/04/15 | 321AC: 2005-04-13 | 321AC:00032883 | E0409 | 321AC WHQL HCT12.0 pass |
| 2005/04/20 | 321AB: 2005-04-19 321AC: 2005-04-20 | 321AB:0002014F 321AC:0003263E | E0420 | - Test Program add recognize USB Hub port function - 321AB fix win2k auto run - 321AC fix run memorybar partition plug-in show security partition |
| 2005/04/21 |  |  | E0421 | 1. twin mode dbf file setting (@ : single mode ! : twin mode) |
| 2005/04/22 |  |  | E0422 | 1. Win98 VIA Config hub problem |
| 2005/04/27 |  |  | E0427 | 1. 321AC after test OK LED flash |
| 2005/04/29 |  |  | E0429 | - add 321acptest.bin (Pretest) - fix 321AC after test LED no flash - test different flash on the same time |
| 2005/05/03 | 323ADISP: 2005-05-02 323ADISP_SEQ: 2005-05-02 | 323AD: 3B4FD 323AD_SEQ: 3B9A1 |  | 1.DevEsti Program can’t detect 323AD |
| 2005/05/04 | 321ACISP: 2005-04-25 | 321AC: 35E96 | E0504 | TestAP: auto scan usb when plug-in 321AC ISP :fix suspend current |
| 2005/05/09 |  |  | E0509 | TestAP: Fix 0504 run 321AB and 320AC show error message |
| 2005/05/12 | 321ACISP: 2005-05-02 | 321AC:3B4FD | E0512 | TestAP: Add Scan show bad block number 321ACISP: fix 321AC+Micro junior fail |
| 2005/05/17 |  |  | E0517 | TestAP:Display show badblock error |
| 2005/05/20 |  |  | E0520 | TestAP: 1. fix multi-device make auto run , only one device have image 2. Remove Win2k unsafe remove message 3. E0517 pretest bad block total over 20 then fail problem |
| 2005/05/24 | 323AD: 2005-03-15 323AD_SEQ: 2005-03-31 | 323ADISP :0003AD79 323ADISP_SEQ: 0003B5E9 |  | E0503 323ADISP Mac 9.2 can’t recognize |
| 2005/05/26 |  |  | E0526 | TestAP: 1. add format retry 2. add erase info block before pretest |
| 2005/06/01 |  |  | E0601 | TestAP: 1. add write protect check before pretest 2. modify 321AC test program check bad block |
| 2005/06/08 |  |  | E0608 | TestAP: 1. add a reset after download ISP , for k9w8g usb 1.1 unformat problem 2. add win98 no config hub can test 3. remove 0x21 vendor command in ReadConfig Button . |
| 2005/06/16 |  |  | E0608 | 1. E0608 add reset make 321AC after test LED no Flash , update ledproc321ac.bin |
| 2005/06/23 |  |  | E0623 | - Check bad block can support MU large then 16 - Hynix 8G twin 323AD , Run E0608 ISP no download |
| 2005/06/24 | 321AC 2005-06-24 | 321AC: 00051C99 | E0624 | - add 321AC partial flash support - 321AC support micron interleave |
| 2005/07/01 | 321AC 2005-07-01 | 321AC: 00050CCA | E0701 | - add a partial flash and normal flash check box , if no check then capacity is calculate by bad block , if check then capacity is calaulate by partial size - 321ac ISP fix partial flash autorun function |
| 2005/07/07 |  |  | E0707 | - add preload include sub directory function - fix Win2000 test multi-device OK , plug-out usb show “Get IC Version Fail” |
| 2005/07/15 | 321AC_NR 2005-07-15 |  | E0715 | 1. add SM321ACISP_NR.BIN isp for hynix NR chip |
| 2005/07/25 |  |  | E0725 | 1. add time out setting |
| 2005/07/27 |  |  | E0727 V2 | - add 321acptest_nr.bin for Hynix NR Pretest - fix 323AD ST Hynix some time copy compare fail |
| 2005/08/02 |  |  | E0802 | 1. add 321AC support Hynix NR ISP |
| 2005/08/04 |  |  | E0804 | - fix Preload file too large problem - add micron partial flash - fix get flash dbf file only get CE0 |
| 2005/08/08 |  |  | E0808 | - modify partial size setting from M bytes to Block - fix 0804 dbf file bug |
| 2005/08/11 |  |  | E0811 | - 321AC If have ISP then pretest fail - add hynix 512 downgrade |
| 2005/08/12 |  |  | E0812 | 1. Update Config Hub function just need one scan then config |
| 2005/08/15 | 321AC 2005-07-01 | 321AC: 0005633B | E0815 | - customer = 1 old memorybar , 8 new memorybar - update 321acisp for old and new memorybar |
| 2005/08/17 | 323AD: 2005-08-17 323AD_SEQ: 2005-08-17 | 323ADISP :0004B7CD 323ADISP_SEQ: 0004CB57 | E0817 | - 321AC K9WAG twin mode pretest fail - 323AD IBM NoteBook suspend resume |
| 2005/08/18 | 323AD: 2005-08-18 323AD_SEQ: 2005-08-18 | 323ADISP: 00050D41 323ADISP_SEQ: 000520CB | E0817 | 1. 323AD IBM NoteBook i1200 cold boot Problem |
| 2005/08/19 | 323AD: 2005-08-19 323AD_SEQ: 2005-08-19 | 323ADISP: 00050EDB 323ADISP_SEQ: 00052C21 | E0817 | 1. 323AD fix chkdsk problem |
| 2005/08/24 |  |  | E0824 | 1. add AGAND 4G support |
| 2005/09/05 |  |  | E0905 | - fix 0812-0824 config hub can’t scan large 2 hub . - add readback test program version . - add when > 2G format to FAT32 , (but preload function only support fat 16) |
| 2005/09/09 |  |  | E0909 | - add Scan capacity function - change partial size setting form block to M byte |
| 2005/09/13 |  |  | E0913 | - add back removeable type have partition table , because linux can’t mount |
| 2005/09/15 |  |  | E0915 | 1. modify test program UI show bmp location |
| 2005/09/16 |  |  | E0916 | - add usb 1.1 button - modify scan capacity |
| 2005/09/21 |  |  | E0921 | 1. modify pretest string “No/RW Test” to “Erase only” |
| 2005/09/22 | 323ad_SEQ: 2005-09-22 | 323adisp_seq: 00053729 | E0921 | - update 323adisp_seq.bin for check sum error problem - update infupdate for no copy file to drivers folder |
| 2005/09/28 |  |  | E0928 | 1. after test ok , unplug 1 usb , don’t refresh test result |
| 2005/10/05 |  |  | E1005 | - modify show icon - modify 321acptest.bin for intlv mode second flash no reference default bad block |
| 2005/10/21 |  |  | E1021 | - update smi32x2k.sys for avoid blue screen - fix preload file cluster different bug |
| 2005/10/25 |  |  | E1025 | - New UI |
| 2005/10/26 |  |  | E1026 | - UI display on 800X600 |
| 2005/10/28 |  |  | E1028 | - UI modify - open down grade setting bad block 44 limit |
| 2005/11/02 |  |  | E1102 | 1. fix E1028 AGAND can’t set BB less then 44 |
| 2005/11/10 |  |  | E1109 | - add 332aa support - add copy test % select - add font size can adjust |
| 2005/11/11 |  |  | E1111 | 1. add downgrade flash auto capacity |
| 2005/11/15 |  |  | E1115 | - add auto install driver - list can zoom to all window |
| 2005/11/18 |  |  | E1118 | 1. fix E1111 downgrade auto capacity |
| 2005/11/30 |  |  | E1130 | - ignoring MU check when downgrade flash - Duplicating write command after DoReset for K9G4G flash to solve LBA0 without FORMAT. - CLIstCtrl Column Show and Hide. - Update Udisk information after test finish. - Add double click in ClistCtrl Item for reading CID information. - Decreasing refresh times when plug or un-plug udisk. - Edited copy compare percentage show when password failed. - Remember last column width in ClistCtrl. - Disable “Open Device Fail” Message. - Disable ClistCtrl column Auto-resize. |
| 2005/12/1 |  |  | E1201 | - Update SM321ACPTEST_micron.bin - Support K9HAG formatting with FAT32.(Label will exist) - Support Autorun+Public+Private 3IN1 option with APP3IN1=1 in INI file. |
| 2005/12/2 |  |  | E1202 | - Update the INF Update program to solve losing detect UFD with factory driver in win2000.(Example: plug 14 UFDs but showed 13 UFD in Device Manager) |
| 2005/12/5 |  |  | E1205 | - Fixed the port # (under item column) become “090c/1000” - Fixed not refresh “final actual” capacity, until 1 UFD is unplugged (or rescan) - Fixed “Open Device Fail” problem. - Add user setting the EndHead, EndSector and End Cylinder information. |
| 2005/12/8 | 321ACISP: 2005-12-08 | 321ACISP: 00057D74 | E1208 | 1. Update the SM321ACISP.BIN for AUTORUN Duplication fixing in WinXP. |
| 2005/12/12 |  |  | E1212 | - Add the setting of the maximum length of serial number in setting dialog. - Update the erasing processing before pretest for K9L8G flash. |
| 2005/12/16 | 321ACISP: 2005-12-16 | 321ACISP: 00057FE0 | E1216 | - Update the SM321ACISP.BIN for Autorun three times in win2k sp4 with usb1.1 - Erasing ISP process before pretest for SM322AD. |
| 2005/12/22 |  |  | E1222 | - Add Serial Number for SM340 - Support SM324BA |
| 2005/12/23 | 324BAISP: 2005-12-23 | 324BAISP: 0012F131 | E1223 | - Update the SM324BAISP.BIN - Update the Checking of CardMode for 512 and 2K |
| 2005/12/28 | 324BAISP: 2005-12-26 | 324BAISP: 00137A03 | E1228 | - Update SM324BAISP.BIN for Read Write Clock separate setting . |
| 2006/01/10 |  |  | F0110 | 1. Modify the pretest string. |
| 2005/12/29 | 321ACIAP: 2005-12-29 | 321ACISP” 0006BCA9 | E1228 | 1. update 321acisp.bin for autorun + memorybar bug. |
| 2006/1/12 | 324BAISP: 2006-01-12 | 324BAISP: 0014D8F2 | F0112 | - Modify the ScanUSB button only if 「102」caption became non zero in string table. - Modify the Serial Number process with mask and auto update with date. |
| 2006/01/13 |  |  | F0113 | 1. If autorun image too large , program will crash . |
| 2006/01/17 | 324BAISP: 2005-01-16 | 324BAISP: 00014D932 | F0117 | - update 324ISP for copy compare fail - update 324 pretest isp - add 2 flash to 321 dbf file |
| 2006/01/19 |  |  | F0119 | - update AGAND downgrade pretest , for some time capacity reduce half - modify check bad block only when do pretest |
| 2006/01/20 |  |  | F0120 | 1. modify sm32xtest2.exe add start button |
| 2006/01/26 |  |  | F0126 | - remove 323AD reset - add infineon 0x66 force ID - preload data problem |
| 2006/01/27 | 321ACK9HAGISP: 2006-01-27 | 321ACK9HAGISP: 00068861 | F0127 | 1. fix K9HAG miss compare problem |
| 2006/02/15 | 321ACPTEST: 2006-02-15 |  | F0215 | - 321AC add hynix MLC pretest check last 2 page default bad block - 324BB add dbf K9F4G K9K8G K9WAG pll setting - 324BB small block flash download isp problem - if no select cid then can’t select auto run |
| 2006/02/16 | 324BBISP: 2006-02-15 324BBPTEST: 2006-02-16 |  | F0215 | - 324BB MLC twin mode support - mp tool show error card mode (mlc , slc), |
| 2006/02/20 | 324BBPTEST: 2006-02-20 |  | F0215 | 1. modify pretest blkn1 ,blkn2 |
| 2006/02/22 | 324BBISP: 2006-02-22 |  | F0215 | 1 . K9K8G add erase cid before pretest 2 . K9K2G copy compare fail. |
| 2006/02/23 | 324BBISP: 2006-02-23 |  | F0215 | - modify 324 dbf k9k8g , k9f4g clock to 0x13 - modify 324bb isp for mlc twin mode |
| 2006/02/24 | 324BBISP: 2006-02-24 |  | F0215 | 1. modify 324BB isp K9L8G 2 plane mode , format ok , power off , then unformat |
| 2006/03/02 |  |  | F0302 | 1. 321BB mp tool load wrong dbf file |
| 2006/03/06 | 324BBISP: 2006-03-05 |  | F0302 | 1. 324BB + K9HAG 2 plane mode , no format problem |
| 2006/03/07 | 321ACK9HAGISP: 2006-03-07 321BBISP: 2006-03-06 324BBISP: 2006-03-07 |  | F0307 | - 321AC K9HAG partial write limit to 1 - 321BB small block usb enum problem - 324BB K9NBG support 4. fat32 change reserve 0x26 to 0x20 |
| 2006/03/08 |  |  | F0308 | - fat16 and fat32 format modify - 321BB+infineon HYF33DS - 321BB+Toshiba TC58NVG1D4B |
| 2006/03/10 |  |  | F0310 | - fat16 and fat32 for mac os 10.1 - 321BB card mode - 321AC downgrade flash check bad block |
| 2006/03/14 | 321ACISP: 2006-03-14 | 321ACISP: 00062048 | F0314 | - 321AC hynix MLC , Samsung MLC . miss compare problem - add 324 ST MLC dbf file |
| 2006/03/15 | 321ACISP: 2006-03-15 | 321ACISP: 000624ED | F0315 | - MP tool format totalsector –1 - fix 0314 321acisp 2K MLC fat have data problem |
| 2006/03/16 | 321BBISP: 2006-03-16 324BBISP: 2006-03-16 | 321BBISP: 0006CE97 324BBISP: 0017F795 | F0316 | - MP tool add every mu copy test - fix 324BB usb 1.1 mp tool pretest fail - fix 324BB and 321BB on 2K MLC fat data problem 4. add 324 0xc1 0x76 force id |
| 2006/03/17 | 324BBISP: 2006-03-17 | 324BBISP: 00018266C | F0317 | - MP too preload long file name - 324BB Mac format , mac extension hang |
| 2006/03/24 | 324BBISP: 2006-03-24 | 324BBISP: 00017B9A3 | F0324 | 1. 324BB ISP fix when read data insert inquiry command then data error problem |
| 2006/03/25 |  |  | F0325 | 1 add SM324BBPTEST_SMIDriver.BIN file for SMI driver pretest fail problem |
| 2006/03/28 |  |  |  | 1.fix 321BB AGAND pretest reference default bad block problem |
| 2006/03/31 | 321BBISP: 2006-0331 324BBISP: 2006-0331 | 321BBISP: 0054144 324BBISP: 017CE47 | F0331 | - 321BBISP , add multi-lun string - 324BBISP , add multi-lun string - 324BBISP-R , for AGAND flash - MP tool , 321BB test pass led flash |
| 2006/4/3 |  |  | F0403 | - Add USBDisk Image for preload function. - Fix AGAND Flash download 16K isp if isp size more than 16k for SM324BB |
| 2006/04/06 | 321BBISP: 2006-0406 | 321BBISP: 0053FB6 |  | 1. 321BBISP for warm boot device not recognize problem |
| 2006/04/07 | 321BBISP: 2006-0407 321AC2KMLCISP: 2006-0407 | 321BBISP: 0005AD44 321AC2KMLCISP: 00063708 | F0406 | - Fix SM321BB reboot issue. - Fix SM321AC miss compare with K9HAG. - Update Flash_321BB.DBF and Falsh_324.DBF - Fix Format function in the MPTool. Let First data sector position in the multiple of 4. |
| 2006/4/10 | 321ACISP: 2006-04-10 321BBISP: 2006-04-10 | 321ACISP: 0006F7FC 321BBISP: 0005ACF8 | F0406 | - Fix the pretest for SM324BB-R. - Add the multi lun string for SM321AC. - Fix SM321BB reboot issue (0407 still fail) |
| 2006/4/12 |  |  | F0412 | - Change the MLC bad block information from last page. - Fixed the SM321BB copy-compare issue when MP tool running |
| 2006/4/20 | 321ACISP 2006-4-20 321BBISP 2006-4-20 | 321ACISP 00060806 321BBISP 00069CF7 | F0420 | - Modify the SM32x DBF file for the timing setting - Insert 0x85 command for the Micron 70nm Flash |
| 2006/4/21 | 323ADISP 2006-4-21 | 323ADISP 00052445 |  | 1 Add the SM323 auto run + memory bar |
| 2006/4/25 | 324BBISP 2006-04-25 | 324BBISP 0019867A | F0425 | - Insert 0x85 command for the Micron 70nm Flash - Preload in FAT32 format - Improve SM324+K9F4G for the performance in 2-plane mo |
| 2006/5/4 | 324BBISP 2006-05-04 321AC2KMLC 2006-04-26 | 324BBISP 00198064 321AC2KMLC 000638A1 | F0504 | - SM321AC ISP fixed auto-run for the 2K MLC in Win2000 - SM324BB ISP fixed the copy-compare failure in WinMe with 2-plane mode ( 2pcs of the K9F4G) - Fixed the automatics turn-off MU option for the Spectek flash - Fixed Win2000 copy-compare error when the MP tool run in the background - Modify the DBF file to correct the Hynix MLC/SLC Flash setting |
| 2006/05/15 | SM321ACISP 2006-05-11 SM321BBISP 2006-05-15 | SM321ACISP 00061B16 SM321BBISP 0006B14F | F0515 | - Adjust the format size for the 128MB and 256MB as the 0315 - Disable MP tool copy-compare when enable the auto-run function - SM321BB randomly copy-compare failure during full size copy and plug-in/plug-out several times - Disable the copy-back function for the Hynix MLC |
| 2006/05/22 |  |  | F0522 | 1. Fixed the capacity inconsistence for the SM321E(SM321BB) |
| 2006/06/15 | SM321BBISP 2006-06-15 SM324BBISP 2006-06-15 SM323ADISP 2006-06-01 | SM321BBISP 00086AA8 SM324BBISP 00198C8A SM323ADISP 00052A96 | F0615 | - SM321BB support the Hynix HY27UV08AG5M interleave mode - Update the DBF file for correcting the flash option - SM324BB fixed the Samsung K9K1G08U0M power-off capacity didn’t match - SM321BB and SM324BB support the Samsung K9G8G - SM323AD correct he product string didn’t show up 6. MP tool correct the FAT32 format as the WinXP cluster size |

| 2006/06/19 | SM321BBISP 2006-06-17 | SM321BBISP 00087027 | F0619 | - SM321BB fixed HY27UV088G5M interleave mode in WHQL Syscache - SM321BB fixed MLC interleave pretest bad block number |
| --- | --- | --- | --- | --- |
| 2006/07/18 | SM324BBISP 2006-07-18 | SM324BBISP 00199C1C | F0718 | - SM324ISP for the Samsung K9F2G08U0A - Improve the SM324 with MLC flash performance - The MP tool fixed the some SM324 controller can’t complete all testing steps in the second time. - Update the dbf files for the new flash |
| 2006/08/21 | SM321ACISP 2006-08-14 SM321BBISP 2006-08-18 SM324BBISP 2006-08-17 | SM321ACISP 0006AD34 SM321BBISP 0008EAF5 SM324BBISP 001D3828 | F0817 | - SM321AC support the Samsung K9K8G interleave mode - SM321AC and SM321BB fix the CD-Creator compatibility - SM321AC support the Samsung K9F2G08U0A and K9LAG - SM321BB support the MSC USBCV r1.3 for the case13 - SM321BB improve the MLC performance in Win2K - SM324BB support the multi-luns function - The MP tool support the CDROM size to 32GB - The MP tool fix the preload problem in FAT32 format |
| 2006/08/30 | SM324BBISP 2006-08-29 | SM324BBISP 001D2F5D | F0830 | - SM324BB add the floppy drive supporting - SM324BB MP tool for the multi-device testing unstable by using factory driver - SM321BB for the ignore “status checking” by the SM321bbisp_2k_nr.ISP - The MP tool add the optional for the FAT32 format - The MP tool add the preload to Floppy drive |
| 2006/09/13 | SM324BBISP 2006-09-13 SM321BBISP 2006-08-17 | SM324BBISP 001D6EBF SM321BBISP 00086D00 | F0913 | - SM324 fix the copy-compare bug for the K9F2G08U0A - SM324 MP tool‘s factory avoid the testing stuck when one device failed - Support the Powerchip P1U1GR30AT - DBF setting for the Intel 72nm Flash: 29F04G08AANB1, 29F08G08BANB1, and 29F08G08FANB3 - The SM321BB remove the case 13 parts |
| 2006/09/29 | SM324BBISP 2006-09-15 SM321BBISP 2006-09-28 | SM324BBISP 001DBA67 SM321BBISP 00086DB3 | F0928 | - Support the AGAND Flash 1GB can be used just half size - SM324 –R ISP for the AGAND capacity increase to 485MB - Support the Specteck auto disable MU - SM321BB ISP fix the interleave mode for the MLC Flash - MP tool add the option for the K9F2GU0A single plan or two plane mode in the default.ini to cover too many bad block# in two plane |
| 2006/10/05 | SM321BBISP 2006-10-05 SM324BCISP 2006-10-04 | SM321BBISP 00086016 SM324BC 0004CC8F | F1005 | - SM321BB fix the MAC 10.4.2 can’t recognized - SM324BC release |
| 2006/10/20 | SM321BBISP 2006-10-20 | SM321BB_2KMLC 000B7401 | F1020 | - SM321BB_2KMLC support the Samsung 63nm MLC (K9G8G, K9LAG, K9HBG, K9MCG) NOP=1 - MP tool fix the AGAND pretest second time failed - MP tool factory driver for the SM324BC |
| 2006/10/25 | SM324BCISP 2006-10-20 | SM324BCISP 0006A6AB | F1025 | - SM324BC pretest will not erase default bad block info - MP tool add flash ID 4bytes info - MP tool reduce the AG-AND pretest time and correct bad block - MP tool extend the CheckSize option from 4GB to 16GB |
| 2006/10/27 | SM321ACISP 2006-10-25 SM321ACISP 2006-10-27 SM324BBISP 2006-10-26 SM324BCISP 2006-10-26 | SM321AC_2KMLC 000BD883 SM321ACISP 0006ADB4 SM324BBISP 001E12AC SM324BCISP 0006AA97 | F1027 | - SM321AC support NOP=1 for the Samsung MLC - SM321AC fix write-protect in the MAC 10.3.x - SM324BC fix the copy-compare error for the K9G8G and K9MCG - SM324BB fix the SM324 LED blanking abnormal status - SM324BC correct the right capacity - MP tool fix the SM321BB for the Micron MLC bad block number - SM324BB – R for the AG-AND CID/ISP corruption |
| 2006/11/03 | SM321ACISP 2006-11-02 SM324BCISP 2006-11-03 | SM321AC_2KMLC 000BD712 SM324BCISP 0006E20F | F1103 | - SM321AC NOP=1 ISP for the memory bar security mode - MP tool fix the SM321AC for the flash bad block info with different card mode - Increase Samsung MLC access cycle time from 50ns to 30ns - SM324BC ISP to increase the small file copying speed. - SM324BC ISP modify the Max. bad block number |
| 2006/11/14 | SM321BBISP 2006-11-13 SM324BCISP 2006-11-13 | SM321BB_2KMLC 000B870C SM324BCISP 000749A4 | F1114 | - MP tool re-organized the subdirectory for the each product - MP tool check the Hynic MLC in the last page or last page –2 - Set the Hynix MLC clock to 0x06 (82ns) - MP tool to display correct bad block number for the SM324BC - SM321BB NOP=1 for mark the bad block for the mode changes of the Samsung MLC - SM324BC max. the bad block number - SM324BC for the Renesas single channel supporting - SM324BC fixed the H/W write protect problem in WinMe and Win98 - SM324BC will not check the RB0/RB1 for the 2-plane mode. |
| 2006/11/29 | SM321ACISP 2006-11-29 SM321BBISP 2006-11-29 SM321BBISP 2006-11-29 SM324BCISP 2006-11-27 | SM321AC_2KMLC 000BDD01 SM321BBISP_2KMLC_MSC 000B1D42 SM321BBISP_2KMLC 000B9080 SM324BCISP 0007E79D | F1129 | - SM321AC NOP=1 ISP fixed the memory bar application problem in Samsung K9G8G - SM32x NOP=1 ISP fixed the power on re-build table - SM324BC fixed the reading of the default bad block - MP tool changed of the SM324BB and SM324BC clock setting - MP tool add the user specific Flash part number option - MP tool correct the SM324BC’s reserved max. bad block to meet the Flash spec. (SLC 2%, MLC 4%, Samsung MLC 2.5%) - MP tool choose the auto run in SM321BB will disable the MSC |
| 2006/12/19 | SM321BBISP 2006-12-15 SM321BBISP 2006-12-15 SM324BBISP 2006-12-15 | SM321BBISP_MSC 0008AF2F SM321BBISP 00084BF0 SM324BBISP 001E1AAF | F1219 | - SM321BB ISP to support the Samsung K9F1G08U0B - SM321BB ISP to support the Vista RTM Auto-run - SM324BB ISP for the Micron/Intel MT29F2G08AAC, MT29F4G08AAA, MT29F4G08BAB for the performance - F1219 MP tool to avoid the SM321BB MSC and non-MSC ISP loading - F1219 MP tool had full save the parameters in INI file with “save as” to user ini file. - F1219 MP tool to support the preload file with the FAT12 - F1219 MP tool to support the user setting for the max. bad block numbers in default.ini by the “original BadBlock=1” - F1219 MP tool to support the Samsung K9F1G08U0B - F1219 MP tool’s Auto-capacity option to have all good blocks - DBF File changes for the SM321AC, SM321BB, SM324BB, and SM324BC - Samsung K9LAG08U0M, K9HBG08U1M, K9F1G08U0B - Hynix HY27UV08AG5M, HY27US08121B - ST NAND08GW3B2AN6, NAND04GW3B2BN6 NAND02GW3B2AN6 - Micron MT29F8G08BAA, MT29F16G08FAA, MT29F8G08FAB, MT29F8G08FAC - Intel JS29F04G08AANB1, JS29F16G08FANB1, JS29F08G08BANB1, JS29F08G08FANB3, JSF32G08FAMB1, JSF16G08CAMB1 JSF08G08AAMB1 |
| 2007/01/23 | SM321BBISP 2007-01-22 SM321BBISP 2007-01-22 SM324BCISP 2007-01-23 SM324BBISP 2007-01-26 | SM321BBISP_2KMLC 000B6CDF SM321BBISP_2KMLC_MSC 000 B795B SM324BCISP 0008F45A SM324BBISP 001E1C8D | G0117 | - SM321BB and SM324BC ISP for the Vista ready Boost read performance - SM324BB ISP fix the Micron SLC flash write timing - SM324BC ISP fixed the Case13 for the IMI tester - SM324BC ISP fixed the WHQL “frame work” - MP tool add option “justscan” in the default.ini for the supervisor mode - MP tool support the random serial number and 13 bytes - MP tool display the % of the total reserved block - MP tool fixed the copy-compare when enabling Floppy mode |
| 2007/02/09 | SM321BBISP 2007-02-02 SM324BCISP 2007-02-09 SM324BBISP 2007-02-09 | SM321BBISP_MSC 00084AE6 SM324BCISP 00098787 SM324BBISP 001E20F0 | G0209 | - Enhance the SM321BB for the SLC ready-boost read performance. - SM321, SM321BB, and SM324BC support the Hynix 60nm MLC NOP=1 - SM324BC ECC over 4-bit will mark bad block - MP tool fixed the preload for the long directory name (>11bytes) - SM324BB fixed LED blink when SCSI 0x25 - DBF add the new flash setting: Hynix HY27UT088G2M, HY27UU08AG5M, Micron MT29F16G08QAA, Samsung K9F1208U0C, K9L8G08U0A, Intel JSF16G08CAMB2, JSF08G08AAMB2, JSF32G08FAMB2 |
| 2007/02/12 |  |  | 2007/02/12 | - MP tool and DBF for Hynix HY27UV08BG5M - Slow the SM321AC clock to 0x04 for the Samsung K9WAG08U1A - Disable the SM321AC copy-back for the Micron 29F08G08CANB1 |
| 2007/03/13 | SM321BBISP 2007-03-13 SM321BBISP 2007-03-13 SM324BCISP 2007-03-07 SM324BCISP 2007-03-07 SM321BCISP 2007-03-09 | SM321BBISP_MSC 000923FD SM321BBISP_2KMLC_MSC 000BB139 SM324BCISP 00099D9B SM324BCISP_HBG 0009C6D8 SM321BCISP 000B4BBE | V1.16.04 | - SM321BB support the Samsung K9F1208U0C - SM324BC set max. bad block to 56 will caused Vista format failed - SM324BC with K9F2GU0A had bad block in 0x7A~0x7E location caused “can’t recognized” - SM321BB and SM324BC improve Vista ready boost first time read performance - SM324BC special ISP for the Samsung HBG to have <100mA operation current - Add SM321BC ISP - MP tool change to date to version number V1.16.04 - MP tool add the “DISKREADONLY” option in the default.ini |
| 2007/04/11 | SM324BCISP 2007-04-10 SM324BCISP 2007-04-04 | SM324BCISP 0009FE02 SM324BCISP_HBG 0009E383 | V1.16.11 | - MP tool support “Erase bad+55/AA option”. - MP tool fixed MAC 10.1.5 with FAT32 format in G0313 - MP tool fixed the Toshiba MLC bad block restoring. - MP tool fixed the each Single plane bad block display caused the MP tool display crash issue. (with K9NBG) - SM324BC ISP add the mark bad block when ECC error and not to disconnect the USB - SM324BC ISP fixed the power on save link to avoid the big file miss compare - SM324BC ISP fixed Micron MT29F8G08MAA mis-compare by repeat bad block |
| 2007/05/07 | SM321BBISP 200-04-23 SM321BBISP 2007-05-07 SM321BBISP 2007-05-07 SM324BCISP 2007-04-20 SM324BCISP 2007-04-11 | SM321BBISP 00090700 SM321BBISP_MLC_2plane 0017C03C SM321BBISP_4K 0018CD58 SM324BCISP 000AB47F SM324BCISP_4K 00181E3A | V1.16.16 | - SM321BB ISP fixed autorun showed “restart” in first time with some PC platforms. - SM321BB ISP add 2-plane mode for the MLC Flash to double the 2-plane MLC performance - SM324BC ISP fixed the bad block repeat marking. - SM324BC ISP fixed the plug-out/plug-in lost 64KB copy-back in the SLC copy-back mode. - SM324BC and SM321BB ISP add the Samsung 4KB/page flash (K9F8G08U0M & K9GAG08U0M) supporting (Beta version) for single channel. - MP tool add the 2-plane mode flash force ID - MP tool check max. reserved bad block number to have enough spare block. - MP tool will erase ISP when ISP download error |
| 2007/06/08 | SM321BBISP 2007-06-08 SM324BCISP 2007-06-04 SM324BCISP 2007-06-07 SM324BCISP 2007-06-07 | SM321BBISP_4K 0018A992 SM324BCISP 000D0C77 SM324BCISP_4K_twin 0017CDE4 SM324BCISP_4K_sinlge 00175094 | V1.16.19 | - MP tool add the Samsung 4KB/page dual channel supporting (K9F8G and K9GAG) - MP tool’s default.ini add 2 options for the wear-leveling count parameters n (count number = nx256). Defaults: ISP count 10K (n=40), and Data Write count 2K (n=8). - SM324BC ISP to support the new wear-leveling - DBF file disable Intel MLC cache mode: JS29F16G08CAMB1 JS29F08G08AAMB1 JS29F16G08CAMB2 JS29F08G08AAMB2 JS29F32G08FAMB1 JS29F32G08FAMB2 - DBF file disable the “read programming status” checking for all MLC in SM324BC - DBF file add new flash setting: K9G2G08U0M, K9L4G08U1M, MT29F4G08AAC, NAND08GW3C2A, NAND16GW3C2A |
| 2007/06/29 | SM324BC 2007-06-29 SM324BC 2007-06-29 SM324BC 2007-06-25 SM321BB 2007-06-26 | SM324BCISP_4K_twin 001A2E25 SM324BCISP_4K_sinlge 00199753 SM324BCISP 000 D3E6F SM321BBISP_2Plane 0018D19E | V1.16.22 | - SM321BB and SM324BC support the Hynix 60nm date code after 726A. the date code between 712A to 726A need to use the other special ISP.(test_321_324_Hynix_60nm_G0625). - SM324BC fixed the mark the same bad block when programming failed - SM324BC fixed the 4KB/page copy-comparison failed in USB1.1 - SM324BC fixed the 2x K9F8G burn-in failed - SM324BC add the new wear-leveling - SM321BB and SM324BC fixed the GPS can’t recognized by command 1A to check length is at dCBWTransferLength, not length in CBWCB - MP tool fixed the bad block information for the force ID mode flash when pretest - MP tool Support the Samsung K9KAG08U0M, K9LBG08U0M, K9WBG08U1M. - DBF add new flash setting: NAND08GW3C2AN1 NAND08GW3C2AN6 NAND16GW3C2AN1 NAND16GW3C2AN6 |
| 2007/07/10 | SM324BC 2007-07-09 SM324BC 2007-07-09 SM321BB 2007-07-10 | SM324BCISP_4K_SINGLE 00199733 SM324BCISP_4K_TWIN 001A2DE7 SM321BBISP_2Plane 0018D2DE | v.1.16.24 | - SM324BC fixed the G0629 USB-IF MSC problem for the Samsung 4KB/page Flash - SM321BB fixed the G0629 bug for the auto-run function in 2-plane mode - MP tool for data area start LBA for the 32x for the Samsung 4KB/page performance - MP tool supports the ST NAND08GW3C2AN |
| 2007/08/02 | SM324BC 2007-08-02 SM324BC 2007-08-02 | SM324BCISP_4K_SINGLE 001C0406 SM324BCISP_4K_TWIN 001CD4D0 | v.1.16.26 | - SM324BC improved the Samsung 4KB ISP wear-leveling - SM324BC fixed the Samsung 4KB power-on-off problem - SM324BC support the external Interleave mode for the Samsung 4KB 2CE Flash (K9WBG, K9HCG) to have better performance - MP tool changed the SM324BC Samsung 4KB capacity to consistence - DBF file add the ST NAND256W3A0AN6 and NAND512W3A2BN6 |
| 2007/08/28 | SM321BB 2007-08-22 SM321BB 2007-08-10 SM321BB 2007-08-22 SM321BB 2007-08-10 SM321BB 2007-08-10 SM321BB 2007-08-22 SM321BB 2007-07-24 SM324BC 2007-08-22 SM324BC 2007-08-22 | SM321BBISP 0009FF77 SM321BBISP_MSC 000975BD SM321BBISP_2KMLC 000BE5BF SM321BBISP_2KMLC_MSC 000C0377 SM321BBISP_4K 0018B3F5 SM321BBISP_2PLANE 0019E32E SM321BBISP_2KMLC_MSC_K9G2G 000BB3AD SM324BCISP_4K_SINGLE 001C5A3D SM324BCISP_4K_TWIN 001D4B7A | v.1.16.29 | - SM321BB ISP fixed WinCE 5.0 un-recognized - SM321BB ISP fixed the mode sense ReadTOC function for the Vista auto-run - SM321BB ISP support the Samsung K9G2G08U0M - SM324BC ISP fixed CMD_21_F0 read the link-table problem - SM324BC ISP improve the performance for the Samsung 4KB flash - MP tool support the Samsung K9G2G08U0M - MP tool support the single-plane bad block number - MP Tools check UFD size up to 64GB |
| 2007/10/12 | 2007-10-01 SM325AB 2007-09-19 SM321BB 2007-09-19 SM321BB 2007-09-19 SM321BB 2007-09-19 SM321BB 2007-09-19 SM321BB 2007-09-19 SM321BB 2007-09-19 SM321BB 2007-10-11 SM324BC | 001D0F1A SM325AB 000A1026 SM321BBISP 000975CD SM321BBISP_MSC 000BF414 SM321BBISP_2KMLC 000C034F SM321BBISP_2KMLC_MSC 0018B45B SM321BBISP_4K 0019F01A SM321BBISP_2PLANE 000BB373 SM321BBISP_2KMLC_MSC_K9G2G 000DA677 SM324BCISP | V1.17.02 | - MP tool support the SM325AB - MP tool add the SM325 wear-leveling 2 option, OPT1EWEARLEVELING=4 (ISP swap every 16000 times) and OPT1FWEARLEVELING=50 (block write count difference below 50) - MP tool changed the SM324BC 2KB capacity - MP tool fixed the bad block counting for the 4KB and 2KB in pretest - SM321BB ISP fixed the 2-lun application (read-only and mass storage) for the ready-only lun copy to the mass storage lun - SM321BB ISP fixed autorun didn’t response in the special HP platform with (0xA8/0xAA SCSI command) - SM324BC ISP fixed autorun in the Win2K - SM324BC ISP fixed 24 hours burn sometimes lost CID table in Micron Flash 2CE interleave mode - SM324BC ISP enable the Micron/Intel 72nm SLC Cache program to improve the performance - DBF disable the Micron MT29F32G08TAA copy back function - DBF add JS29F04G08AANC1, JS29F16G08AAMC1, JS29F32G08CAMC1, JS29F64G08FAMC1, K9G8G08U0A, HY27UF084G2B, HY27UT088G2A HY27UF08BGFB, HY27UH08AG5B HY27UF084G2B, HY27UG088G5B |
| 2007/10/31 | SM325AC 2007-10-24 SM324BC 2007-10-24 | SM325ACISP 001E01C5 SM324BCISP 000DA67B | V1.17.05 | - MP tool change the SM325AB/SM321CB to SM325AC/SM321CC - MP tool support the SM325AC/SM321CC oscillator clock setting - SM321CC/SM325AC ISP support the USB-IF - SM324BC ISP corrected the 2KB/p and 4KB/p capacity inconsistence problem. - SM321CC/SM325 AC ISP lower the suspend current - SM325DBF file modify the clock and duty cycle - SM325DBF change max. clock to the “0B” |
| 2007/11/08 | SM325AC 2007-11-08 | SM325ACISP 0001E7A06 | V1.17.06 | - MP tool fixed the capacity will be half when the bad block equaled the max. bad block setting (example, SLC=20, MLC=44) - SM325 ISP fixed the 4CE Flash pretest failed - SM325 ISP support the read and write clock separated setting - SM325DBF change the clock setting format for the new MP tool/ISP - DBF added the Hynix HY27UF082G2B |
| 2007/11/28 | SM324BC 2007-11-27 SM324BC 2007-11-27 | SM324BCISP_4K_SINGLE 0001C5DA4 SM324BCISP_4K_TWIN 0001D4EE1 | V1.17.08 | - SM324BC 4KB ISP fixed USB can’t recognized when 0x1F9 is bad block. - MP tool fixed the SM321CC/SM325AC “disk size” checking option and ISP checksum - MP tool supported the Samsung K9LBG08U1M, and K9HCG08U5M - DBF file Modify Intel JS29F04G08AANB1, JS29F08G08CANB1, JS29F08G08BANB1, JS29F16G08FANB1, JS29F08G08AAMC1, JS29F04G08AANC1 R/W 60Mhz - DBF file of the SM325AC add PFC A1U8GA30GT - DBF file of the SM321BB and SM324BC support the HY27UF082G2B with disable cache& copyback and 60Mhz - DBF file add new K9LBG08U1M, K9HCG08U5M, MT29F2G08AAD, |
| 2007/12/15 | SM325AC 2007-12-14 SM324BC 2007-12-06 SM324BC 2007-12-06 | SM325ACISP 002FECF8 SM324BCISP_4K_SINGLE 0024ADCA SM324BCISP_4K_TWIN 00236ECB | V1.17.12 | - SM325AC ISP support the Micron/Intel/Toshiba 4KB MLC in Twin-2plane mode with clock 0x04 (48Mhz) - SM321CC/SM325AC ISP fixed the nVidia GEFORCE 6100-405(MCP615) USB compatibility - SM324BC 4KB ISP improve the small file wear-leveling - SM324BC 4KB ISP fixed the USB disconnection randomly when ECC error handling - MP tool fixed the SM321CC/SM325AC pretest in “erase bad block with 55 AA” function to mark the bad blocks - MP tool format adjust the SM321CC/SM325AC FAT32/FAT16 alignment - MP tool fixed the turn on/off MU setting - MP tool support the DBF setting “@” for the single channel and “!” for the dual channel - MP tool fixed the “preload” function with the wrong file attribute after preload to UFD |
| 2007/12/19 | SM325AC 2007-12-19 SM321BB 2007-12-14 | SM325ACISP 002FEBE4 SM321BBISP_2PLANE 0001A53C6 | V1.17.13 | - SM325AC fixed LED lighting abnormal with setting after read/write randomly - SM325AC ISP lower suspend current by setting /WP to low when suspend mode - SM325 ISP improve inrush current by delaying the Flash access to 100ms when power-on - SM321BB ISP fixed the burn-in failed with Micron 29F088GMAA by doing the ISP wear-leveling - MP tool solve the format problem to cause the SM325 performance drop in G1215 - MP tool fixed the BIOS choose UFD will frozen in Pheonx ROO7OJ4 - SM325AC DBF correct the Micron/Intel 4KB MLC/SLC in Twin-2plane mode with clock 0x04 (48Mhz) - DBF file add ST NAND02GW3B2DN6, NAND08GW3C2AN6, NAND16GW3C2AN6, HYNIX HY27UF08BGFB, HY27UH08AG5B, MICRON MT29F8G08AAA, MT29F16G08DAA, MT29F32G08FAA |